M X D - User's Guide

M   X   D

M i x e d     d a t a     e X e c u t i v e

f o r

C r y s t a l l o g r a p h i c .

Version 3.5.A - Octobre 1989

Pierre Wolfers

Laboratoire de Cristallographie

C.N.R.S. 166 X 38042 GRENOBLE-CEDEX, FRANCE.

MXD Crystallographic System User's Guide                                  Page 2

Preface

The MXD Crystallographic System is the result of a new approach for   structure determination. The modern program language as PASCAL and ADA are suitable to perform an efficient data syntax analysis, therefore the tradition i to write the PASCAL compiler in PASCAL. The present software is written in PASCAL to take advantage of the dynamic memory allocation, recursive procedures or functions, pointed variable.

This crystallographic system is well suited to use a large variety of linear  or non linear constraints. It can be used for many radiation data together, as Neutron and X-Ray data. The scope is :

Crystallographic and/or magnetic structures, with or without  structure modulation and mono or multi wave-vector modes, and with powder or single crystal data.

The experimental data must be in integrated form, and for powders a management procedure for  a group of overlaping reflection is included. A limited data reduction is possible by the system (as 5 or 3  points integration method).

This crystallographic system has two components:

The MXD program which is the DATA COMPILER, this is the most complex part of the system. This data compiler program is the MXDCMP program which must run in first.

The APPLICATION PROGRAMS which perform the desired computing with the MXD compiled code. The main application program is the Least Square Fitting program MXDLSQ, but some other one can be added in the future.

The MXD REFERENCE is :

MXD: a general least-squares program for non-standard crystallographic refinements.

By P. Wolfers,

Laboratoire de cristallographie, associé à l'Université J. Fourier,

C. N. R. S., 25 avenue des Martyrs,

B.P. 166 X, 38042 Grenoble CEDEX, France.

Reference paper : P. Wolfers, J. Appl. Cryst, 23, 554-557, (1990).

MXD Crystallographic System User's Guide                        Page 3

CONTENTS

CHAPTER I       INTRODUCTION TO MXD.

I.1     GETTING STARTED WITH MXD.  . . . . . . . . . . . . I-2

I.1.1     MXD To Call The MXDCMP Compiler And Perform Some

Various Task.  . . . . . . . . . . . . . . . . . I-2

I.1.2     MLS To Perform A Least Square Run. . . . . . . . I-3

I.1.3     MXDDEL To Reset MXD To Normal State. . . . . . . I-3

I.2     CRYSTALLOGRAPHIC STRUCTURE EXAMPLES. . . . . . . . I-4

I.3     MAGNETIC STRUCTURE EXAMPLES. . . . . . . . . . . . I-5

I.4     TEMPLATE OF FITTING WITH HETEROGENEOUS DATA (X-RAY

AND NEUTRON).  . . . . . . . . . . . . . . . . . . I-8

I.5     TEMPLATE FOR TWIN OR MAGNETIC DOMAIN MANAGEMENT. . I-9

I.6     HELIMAGNETIC STRUCTURE MANAGEMENT. . . . . . . .  I-10

I.7     BECKER AND COPPENS EXTINCTION MANAGEMENT.  . . .  I-11

I.8     POLARIZED NEUTRONS MANAGEMENT. . . . . . . . . .  I-12

CHAPTER II      THEORETICAL MXD BACKGROUND.

II.1    ATOMIC STRUCTURE FACTOR SUMMATION CONVENTION.  .  II-1

II.2    ATOMIC STRUCTURE FACTOR. . . . . . . . . . . . .  II-2

II.3    MAGNETIC STRUCTURE FACTOR. . . . . . . . . . . .  II-3

II.4    POLARIZED NEUTRON INTENSITIES. . . . . . . . . .  II-4

II.5    MODULATED ATOMIC STRUCTURE MANAGEMENT. . . . . .  II-5

II.6    CALCULATED AND OBSERVED VALUES.  . . . . . . . .  II-6

II.6.1    Intensities. . . . . . . . . . . . . . . . . .  II-7

II.6.2    F2 Observation Data. . . . . . . . . . . . . .  II-7

II.6.3    Structure Factor Based Observation Data. . . .  II-8

II.6.4    Polarized Neutron Case.  . . . . . . . . . . .  II-8

II.7    LEAST SQUARE FORMULAE. . . . . . . . . . . . . .  II-9

CHAPTER III     SYNTAX ELEMENTS OF THE MXD LANGUAGE.

III.1   KEYWORD NOTATION.  . . . . . . . . . . . . . . . III-1

III.2   PRIMITIVE PARAMETER NOTATION.  . . . . . . . . . III-1

III.2.1   Numeric Expression Notation. . . . . . . . . . III-1

III.2.2   String Constant Expression Notation. . . . . . III-2

III.2.3   Identifier Notation. . . . . . . . . . . . . . III-2

III.2.4   Expression Operators.  . . . . . . . . . . . . III-3

III.3   INPUT DATA FORM. . . . . . . . . . . . . . . . . III-3

III.4   OPTIONAL AND OBLIGED PRIMITIVE PARAMETERS. . . . III-3

III.5   ALTERNATIVE CHOICE NOTATION. . . . . . . . . . . III-4

III.6   GENERAL MXD STATEMENT FORM.  . . . . . . . . . . III-4

MXD Crystallographic System User's Guide                        Page 4

CHAPTER IV      DATA MANAGEMENT PRIMITIVES OF MXD.

IV.1    HEADING STATEMENT. . . . . . . . . . . . . . . .  IV-1

IV.2    THE PRAGMA STATEMENT.  . . . . . . . . . . . . .  IV-1

IV.2.1    The L Option For List On Or Off. . . . . . . .  IV-2

IV.2.2    The D Option For Data Interpretation Listing.   IV-3

IV.2.3    The M Option For Macro Expanssion Listing. . .  IV-3

IV.2.4    The N Option For Not Existing File Error

Handling.  . . . . . . . . . . . . . . . . . .  IV-3

IV.2.5    The F Option For Source Input Format.  . . . .  IV-3

IV.3    INCLUDE AND CHAINE STATEMENT.  . . . . . . . . .  IV-4

IV.4    ENDFILE STATEMENT. . . . . . . . . . . . . . . .  IV-4

IV.5    THE MXD SOURCE INPUT FILE LOGIC. . . . . . . . .  IV-5

IV.6    THE APPLICATION PROGRAM OPTION SPECIFICATIONS. .  IV-6

CHAPTER V       PHYSICAL PRIMITIVES OF MXD.

V.1     THE UNIT CELL DEFINITION STATEMENT.  . . . . . . . V-1

V.2     DECLARATIONS AND ASSIGNEMENT STATEMENTS. . . . . . V-2

V.2.1     Declaration And Assignement Of Variable. . . . . V-3

V.2.2     Declaration And Assignation Of Parameters. . . . V-3

V.3     VARIABLE MANAGEMENT STATEMENTS.  . . . . . . . . . V-5

V.3.1     FIXED And UNFIXED Statements.  . . . . . . . . . V-5

V.3.2     Variable Range Specification.  . . . . . . . . . V-5

V.3.3     Diagonal Block For Least-square Program. . . . . V-6

V.4     THE ATOM DECLARATION STATEMENT.  . . . . . . . . . V-7

V.5     THE WAVE VECTOR DECLARATION STATEMENT. . . . . . . V-8

V.6     THE POLARIZED DIRECTION DECLARATION STATEMENT. . . V-8

V.7     THE MAGNETIC MOMENT DECLARATION STATEMENT. . . . . V-8

V.8     THE MODULATED DISPLACEMENT DECLARATION STATEMENT.  V-9

CHAPTER VI      SYMMETRY MANAGEMENT STATEMENTS.

VI.1    THE LATTICE STATEMENT. . . . . . . . . . . . . .  VI-1

VI.2    THE CENTER STATEMENT.  . . . . . . . . . . . . .  VI-2

VI.3    THE SYMTRY STATEMENT.  . . . . . . . . . . . . .  VI-2

VI.4    THE GENSPACE STATEMENT.  . . . . . . . . . . . .  VI-2

VI.5    SOME CONSIDERATIONS ON THE USE OF SYMMETRY

INFORMATIONS.  . . . . . . . . . . . . . . . . .  VI-3

CHAPTER VII     THE EXPERIMENTAL DATA MANAGEMENT STATEMENTS.

VII.1   DATA COLLECTION DEFINITION STATEMENT.  . . . . . VII-1

VII.2   DATA COLLECTION REFERENCE STATEMENT. . . . . . . VII-6

VII.3   CLRDATA STATEMENT. . . . . . . . . . . . . . . . VII-6

VII.4   MANAGEMENT OF FORM FACTOR TABLE IN BINARY DATA

FILE.  . . . . . . . . . . . . . . . . . . . . . VII-7

VII.5   MAGNETIC SPECIFICATION STATEMENT.  . . . . . . . VII-7

VII.6   WARNING FOR PREVIOUSLY DEFINED DATA COLLECTIONS. VII-8

MXD Crystallographic System User's Guide                        Page 5

CHAPTER VIII    EXPRESSIONS AND SYMBOLS IN MXD.

VIII.1  THE MXD FUNCTIONS. . . . . . . . . . . . . . .  VIII-3

VIII.1.1  Standard Function Of MXD.  . . . . . . . . .  VIII-3

VIII.1.2  User Defined MXD Function. . . . . . . . . .  VIII-5

VIII.2  RESERVED AND PREDEFINED SYMBOLS. . . . . . . .  VIII-6

VIII.2.1  Predefined Variable Parameters.  . . . . . .  VIII-6

VIII.2.2  Predefined Constant Parameters.  . . . . . .  VIII-7

VIII.2.3  Reserved Parameter Names.  . . . . . . . . .  VIII-8

CHAPTER IX      MACRO FACILITIES IN THE MXD LANGUAGE.

IX.1    MACRO STATEMENTS.  . . . . . . . . . . . . . . .  IX-1

IX.2    LOCAL SYMBOL DEFINITION. . . . . . . . . . . . .  IX-2

IX.3    CONDITIONAL COMPILATION OF A PART OF MXD SOURCE.  IX-3

IX.4    THE REPEAT STATEMENT.  . . . . . . . . . . . . .  IX-4

IX.5    THE STOP ON DETECTED ERROR STATEMENT.  . . . . .  IX-4

IX.6    THE MACRO LIBRARY MANAGEMENT OF MXDCMP.  . . . .  IX-4

IX.7    THE CONTROLLED INPUT/OUTPUT OF MXDCMP. . . . . .  IX-6

IX.7.1    The Controlled Terminal Operations.  . . . . .  IX-6

IX.7.2    The Listing Outputs. . . . . . . . . . . . . .  IX-7

IX.7.3    The User File Management.  . . . . . . . . . .  IX-7

CHAPTER X       THE LEAST SQUARE PROGRAM MXDLSQ.

X.1     THE SPECIFICATIONS OPTIONS.  . . . . . . . . . . . X-1

X.2     THE LISTING OUTPUT OPTIONS.  . . . . . . . . . . . X-3

X.3     OUTPUT OPTIONS TO GO TO OTHER PROGRAMS.  . . . . . X-7

CHAPTER XI      VAX/VMS IMPLEMENTATION OF MXD.

XI.1    MXD INSTALLATION.  . . . . . . . . . . . . . . .  XI-1

XI.1.1    The First MXD Installation Is Performed In

Steps As Below : . . . . . . . . . . . . . . .  XI-1

XI.1.2    TO Upgrade From A Previous Version 3.4 Or

Above. . . . . . . . . . . . . . . . . . . . .  XI-2

XI.2    VAX/VMS MXD COMMANDS.  . . . . . . . . . . . . .  XI-3

XI.2.1    The Least Square Command MLS.  . . . . . . . .  XI-3

XI.2.2    The MXD Command. . . . . . . . . . . . . . . .  XI-4

XI.2.3    The MXDDEL Statement.  . . . . . . . . . . . .  XI-5

APPENDIX A      ERROR NUMBER SUMMARY

APPENDIX B      HOW WRITE MXD DATA AS STANDARD.

MXD Crystallographic System User's Guide                        Page 6

APPENDIX C      LEGAL EXPRESSION OPERATORS AND STANDARD FUNCTION.

C.1     OPERATORS FOR ALL CONSTANT EXPRESSIONS.  . . . . . C-1

C.2     OPERATORS FOR NUMERIC ALL EXPRESSIONS. . . . . . . C-1

C.3     OPERATOR FOR STRING EXPRESSIONS. . . . . . . . . . C-1

C.4     FUNCTIONS FOR VARIABLE EXPRESSIONS.  . . . . . . . C-2

C.5     FUNCTIONS FOR ALL NUMERIC EXPRESSIONS. . . . . . . C-2

C.6     OPERATORS FOR ALL NUMERIC EXPRESSIONS. . . . . . . C-2

C.7     FUNCTIONS FOR NUMERIC CONSTANT EXPRESSION. . . . . C-2

C.8     OPERATORS AND FUNCTIONS FOR STRING EXPRESSION. . . C-3

C.9     THE PRE-DECLARED VARIABLE PARAMETERS.  . . . . . . C-3

APPENDIX D      THE STANDARD MACRO LIBRARIES.

D.1     THE GENERAL PURPOSE MACRO LIBRARY "MXDLIB.MXL" . . D-1

D.1.1     The HELMOM Macro For Helimagnetic Moment

Description. . . . . . . . . . . . . . . . . . . D-2

D.1.2     The HELDSP Macro For Modulated Displacement

Description. . . . . . . . . . . . . . . . . . . D-2

D.1.3     Le Page And E.J. Gabe Extinction Correction. . . D-3

D.1.4     SHEL-X Extinction Correction.  . . . . . . . . . D-4

D.1.5     Cooper And Roose Formula For A Cylinder

Absorption.  . . . . . . . . . . . . . . . . . . D-4

D.2     THE P. BECKER AND P. COPPENS ( LINEX LIKE )

EXTINCTION CORRECTION. . . . . . . . . . . . . . . D-5

APPENDIX E      MXD DATA FILES STRUCTURES

APPENDIX F      MXD INTERFACE WITH OTHER CRYSTALLOGRAPHIC SYSTEMS.

F.1     OUTPUT INTERFACES TO THE CAMBRIDGE LIBRARY MK3.  . F-1

F.2     INPUT INTERFACE FROM CAD4 GONIOMETER (ENRAF NONIUS

4-CIRCLES).  . . . . . . . . . . . . . . . . . . . F-1

F.3     INPUT INTERFACE FROM THE SDP (B. FRENZ)

CRYSTALLOGRAPHIC SYSTEM. . . . . . . . . . . . . . F-1

F.4     OUTPUT INTERFACE TO THE SDP (B. FRENZ)

CRYSTALLOGRAPHIC SYSTEM. . . . . . . . . . . . . . F-2

APPENDIX G      EQUIVALENT REFLECTIONS UTILITIES.

G.1     SORTING AN ASCII FILE IN INCREASING

SIN(THETA)/LAMBDA. . . . . . . . . . . . . . . . . G-1

G.2     EQUIVALENT AVERAGE AND OR TWIN/MAGNETIC DOMAINS

EXPANSSION.  . . . . . . . . . . . . . . . . . . . G-1

CHAPTER I

INTRODUCTION TO MXD.

This chapter is a description of the main aspects of the data file  for

the  MXD system.  It gives also the basic features to use MXD on the VAX/VMS

systems, for the others MXD implantations, some differences can be observed.

The chapter XI gives all the standard implementations details.

The input data files of the Data Compiler (MXDCMP) are text files which

can  be  written  by  a standard text editor ( for example EVE or EDT on the

VAX/VMS systems ).

We will develop with some data examples, the main aspects  of  the  MXD

data, and explain the logical data design of MXD.

For each example, the presentation of the data file model  is  made  in

two  parts:  the left part is the data text and the right part is a comment.

This right part is (as a legal comment in true MXD data) surrounded  by  the

symbol "(*" and "*) or "{" and "}" as this:

(* this is a comment *)

{ this is an other comment }

(* This { is a unique } comment *)

In the following examples, the data file model is  a  template,  not  a

ready-to-use  file.   Thus,  the  reader can understand the structure of the

data for a particular application.

For all magnetic structure problem the user must know the working space

definition  that  is  used  for  the  magnetic  moment components.  The CELL

statement defines the working space for all crystallographic system as :

->   -> ->             -> ->    ->                      ->   ->

x // a, y in the plane a, b and z the direct product of x by y

->  ->

denoted as ( or x o y ).

INTRODUCTION TO MXD.                                                    Page I-2

When the RCELL unit cell statement is used, the defintion is changed to

->  ->  ->                    ->  ->  ->  ->      ->  ->

x + y + z  // (111) axis and  x o a , y o b , and z o c in the plane[111].

The advantage of that last working space definition is  to  keep  the  three

fold  axis  in  the (111) direction, which is more convenient to express the

symmetry relations.

I.1  GETTING STARTED WITH MXD.

The next command can be system dependant, so the  command  to  activate

MXD  can  present  some  difference among the different MXD implementations.

The described commands are the  standard  VAX/VMS  implementation  commands.

The chapter XI gives more information on the supported implementations.

The first thing to use MXD is to execute the MXDDEF  system  procedure.

This  file  (MXDDEF.COM) which is located in a system manager dependant disk

area, is frequently actived by the initial user login command procedure.  It

uses a monitor command line as :

$ @SYS$USERSOFT:[CRYSTALLO.MXD]MXDDEF ! Install MXD definitions.

The correct equivalent line can be inserted in the initial  login  file

(LOGIN.COM).   It  defines the MXD commands and the logical device "MXDLIB:"

that is the MXD software location in the system.

The MXD commands (for VAX/VMS systems ) are :

I.1.1  MXD To Call The MXDCMP Compiler And Perform Some Various Task.

For example, to create the absorption  table  for  a  spherical  single

crystal, you can type in :

$ MXD MXDLIB:ABSORB

... give the required values ...

$

or also to perform a particular computing (If you had lost your  HP  or

TEXAS pocket computer):

INTRODUCTION TO MXD.                                                    Page I-3

$ MXD

MXD> DISPLAY ' 2(1/2) = ', 2**(1/2);

2(1/2) =  1.414213538E+00

MXD>

$

I.1.2  MLS To Perform A Least Square Run.

This command can receive three parameters.

The first one is the MXD file specification that you  want  to  submit,

that is always required but as a defaulted file type ".MXD".

The second one is the listing device that is defaulted  to  the  user's

terminal.  The "*" listing device is equivalent to TT:  the user's terminal.

The third one is the new  save  device  assignement  (for  the  logical

device  "IN:").   This  device can be used to reset the fit parameters ( The

MXD VARIABLES ) at the values computed from a  previous  least  square  run.

The default value of this parameter is the null device (NL:) after the login

(MXDDEF execution) and the value is not changed when this parameter  is  not

given.  To restart from the initial state the user must specify "NL:" and to

restart from last state the user can specify "SY:" that is equivalent to the

current default directory.

For example if the MXD file has the name "UAPHA.MXD" :

To write listing on the terminal :

$ MLS UALPHA

To continue the least square from the last MXD variable state :

$ MLS UALPHA * SY:

To restart the least square from the initial state

and generate a listing to the line printer $PRINTER (that must be

defined):

$ MLS UALPHA $PRINTER: NL:

I.1.3  MXDDEL To Reset MXD To Normal State.

After a crash or user stop  by  the  CTRL  Y  key  MXDDEL  deletes  all

trailing  initialization  MXD files with the name "MXD.INI".  MXDDEL must be

used after any MXD stop before the normal end, especially when you  want  to

use the MXD command without parameters.

INTRODUCTION TO MXD.                                                    Page I-4

I.2  CRYSTALLOGRAPHIC STRUCTURE EXAMPLES.

This is a small example easy to understand just  given  to  demonstrate

the  MXD  principles  in  a  very simple case.  This is the case of a powder

neutron diffraction patern without overlapping reflection  (all  reflections

are  separately  integrated)  with  nuclear  diffraction  only  (no magnetic

contribution to the reflections).

OPTION(NCYCLE)=2;                     (* WE RUN MXDLSQ FOR 2 CYCLES *)

OPTION(LISTHKL)=ALWAYS;               (* WE PRINT REFLECTION LIST AFTER THE

LAST CYCLE *)

OPTION(LISTMAT)=YES;                  (* WE PRINT ALSO THE CORRELATION

MATRIX *)

TITLE ' U ALPHA ESSAI DE MDX-MDXLSQ ';(* GIVE A TITLE FOR THIS RUN *)

{ UNIT CELL }                         (* ALTERNATE FORM FOR DATA COMMENT *)

CELL 2.884,5.869,4.932;               (* UNIT CELL - ORTHORHOMBIC SYSTEM *)

{ SPACE GROUP ELEMENT FOR CMCM }

CENTER;                               (* THE SPACE GROUP IS CENTERED *)

SYMTRY('E')     = 1,  0,  0,  0.0,    (* IDENTITY SYMMETRY ELEMENT *)

0,  1,  0,  0.0,

0,  0,  1,  0.0;

SYMTRY('2X')    = 1,  0,  0,  0.0,    (* BINARY 2x AXIS *)

0, -1,  0,  0.0,

0,  0, -1,  0.0;

SYMTRY('2Y')    =-1,  0,  0,  0.0,    (* BINARY 2y AXIS *)

0,  1,  0,  0.0,

0,  0, -1,  1/2;

SYMTRY('2.1Z')  =-1,  0,  0,  0.0,    (* BINARY HELICOIDAL 2z AXIS *)

0, -1,  0,  0.0,

0,  0,  1,  1/2;

{ VARIABLE DEFINITION }

VARIABLE SCALE=135.68;                (* DEFINE SCALE AS A VARIABLE

WITH AN INITIAL VALUE OF

153.68 *)

LSQBLOCK('SCALE')=1;                  (* THE SCALE VARIABLE IS

INDEPENDENTLY REFINED IN

A LEAST-SQUARE DIAGONAL BLOCK

WITH A DAMPENING FACTOR OF 1 *)

VARIABLE UY=0.1019;                   (* DEFINE UY AS VARIABLE WITH AN

INITIAL VALUE OF 0.1019 *)

VARIABLE BU = 0.4296;                 (* DEFINE BU AS A VARIABLE WITH AN

INITIAL VALUE OF

0.4296 = DEBYE PAR OF URANIUM *)

PARAM UU = BU/(8*3.1416**2);          (* DEFINE UU AS THE BU EQUIVALENCE

IN U DEBYE FACTOR TERM, UU IS

INTRODUCTION TO MXD.                                                    Page I-5

A VARIABLE PARAMETER NOT A

CONSTANT BECAUSE SOME

VARIABLE (HERE BU) APPEARS IN

THE UU DEFINITION *)

{ NEUTRON DIFFUSION - OR FERMI LENGTH }

PARAM UDIF = 0.853;                   (* DEFINES THE UDIF CONSTANT *)

{ ATOM DESCRIPTION }                  (* DEFINES SOME OTHER CONSTANTS *)

PARAM POP = 1{POPULATION = 1} *       (* A COMMENT CAN BE ADDED ANYWHERE

(1/4) {MULTIPLICITY OF SITE};      A SPACE IS AVAILABLE *)

ATOM('U')=UDIF,,POP, 0, UY, 1/4,UU ;  (* DEFINE AN ATOM WITH THE

NAME "U", THE UDIF REAL SCATTERING

FACTOR, THE CONTRACTED OCCUPANCY

FACTOR OF POP, THE COORDINATES

(0, UY, 1/4), AND THE THERMAL

FACTOR UU. *)

{DATA COLLECTION}

DATA('SILOE-M',SCALE,WE=1/SG)         (* DEFINE A DATA COLLECTION WITH

THE NAME "SILOE-M",

THE SCALE FACTOR SCALE, AND

WHERE THE REFLECTION WEIGHTS

ARE COMPUTED AS 1/SIGMA *)

IH,IK,IL,MUL,RAY,SG;        (* THE DATA FIELD ORDER IS

THE REFLECTION NUMBERS H,K,L,

THE MULTIPLICITY,

THE REFLECTION INTENSITY AND

ITS SIGMA *)

0   2   0   2     61.5    0.4      (* THE DATA COLLECTION FOLLOWS *)

1   1   0   4   1110.2   18.4

0   2   1   4   1189.9   18.4

0   0   2   2    650.8   20.2

1   1   1   8   1097.4    5.0

0   2   2   4    111.5    0.9

END;                                  (* END OF DATA COLLECTION *)

(* END OF MXD DATA FILE *)

I.3  MAGNETIC STRUCTURE EXAMPLES.

This example demonstrates the magnetic structure approach of MXD  in  a

simple case with a triangular structure.

INTRODUCTION TO MXD.                                                    Page I-6

OPTION(NCYCLE)=5;                     (* WE RUN MXDLSQ FOR 5 CYCLES *)

OPTION(LISTHKL)=YES;                  (* WE PRINT REFLECTION LIST AFTER THE

LAST CYCLE *)

OPTION(MXCORREL)=0.65;                (* WE PRINT ALSO THE CORRELATION

MATRIX *)

TITLE ' Example for Magnetic Structures Fitting On Fe2 As';

(* GIVE A TITLE FOR THIS RUN *)

{ UNIT CELL }                         (* ALTERNATE FORM FOR DATA COMMENT *)

CELL 6.490,, 3.597,,,120;             (* UNIT CELL - HEXAGONAL SYSTEM *)

{ GROUPE D'ESPACE P-6 2 M }

LATTICE 'P';                          (* PRIMITIVE LATTICE *)

SYMTRY('-6')  =  0, -1,  0,   0,      (* -6 OPERATOR *)

1, -1,  0,   0,

0,  0, -1,   0;

SYMTRY('MU')  =  0,  1,  0,   0,      (* (110) Mirror *)

1,  0,  0,   0,

0,  0,  1,   0;

GENSPACE;                             (* ASK FOR THE AUTOMATIC SPACE GROUP

GENERATION *)

{ VARIABLE DEFINITIONS }

VARIABLE SCALE=1.8;                   (* DEFINE SCALE AS A VARIABLE

WITH THE INITIAL VALUE OF

1.8 *)

VARIABLE XFE1  = 0.59385,             (* DEFINE THE ATOMIC POSITIONS *)

XFE2  = 0.25621,             (* VARIABLES *)

XAS1  = 0.01523;

VARIABLE UFE1  = 0.005, UFE2  = 0.005,(* DEFINE THE ISOTROPIC   *)

UAS1  = 0.010, UAS2  = 0.004;(* THERMAL VIBRATION VAR. *)

LIMITS 0,, UFE1, UFE2, UAS1, UAS2;    (* LIMIT THE U TO POSITIVE VALUES *)

VARIABLE MFEX = 2.56 { IN Bmu },      (* INITIAL MAGNETIC MOMENT *)

MFEY = 0.6  { In Bmu };      (* OF EACH ATOM *)

LSQBLOCK('GENERAL')= 0.75;            (* SET THE GENERAL DAMPENING

FACTOR TO 0.7 *)

{ FIXED/UNFIXED }

UNFIXED SCALE;                        (* TO FIX SOME VARIABLE Y *)

(* CAN REPLACE "UN" BY "  " *)

INTRODUCTION TO MXD.                                                    Page I-7

UNFIXED XFE1, XFE2;                   (* IN THE APPROPRIATE LINE *)

UNFIXED UFE1, UFE2,UAS1,UAS2;

UNFIXED MFE1, MFE2;

{ READ THE LAST FITTING RESULT }

INUCLUDE 'IN:M.SAV', L:2;              (* THE L:2 OPTION SET THE LISTING

OF THE READ VARIABLE VALUE *)

{ MAGNETIC FORM FACTOR }

(*

It can be included from an other file as this INCLUDE "'FE.FRM';" or

as here directly defined in the MXD data file (so called source file)

*)

PARAM FEFRM(0.0,0.05) = (1, { the 1/2d step is 0.05 and origin is 0 }

.965,  0.900,  0.806,  0.693,  0.574,  0.460,  0.358,

0.274,  0.206,  0.153,  0.111,  0.079,  0.053,  0.033,

0.017,  0.006, -0.002, -0.006, -0.009, -0.010 );

{ ATOMIC STRUCTURE DESCRIPTION }

{ THE MAGNETIC ATOMS ARE DESCRIBED IN THE P1 GROUP }

ATOM('FE11')  = BFE,,   1,  XFE1,     0, 1/2, UFE1;

ATOM('FE12')  = BFE,,   1,     0,  XFE1, 1/2, UFE1;

ATOM('FE13')  = BFE,,   1, -XFE1, -XFE1, 1/2, UFE1;

ATOM('FE21')  = BFE,,   1,  XFE2,     0,   0, UFE2;

ATOM('FE21')  = BFE,,   1,     0,  XFE2,   0, UFE2;

ATOM('FE21')  = BFE,,   1, -XFE2, -XFE2,   0, UFE2;

{ THE NON MAGNETIC ATOMS ARE DESCRIBED IN P-62M GROUP }

ATOM('AS1')    = BAS,,1/12,     0,     0, 1/2, UAS1;

ATOM('AS2')    = BAS,, 1/6,   1/3,   2/3,   0, UAS2;

{ MAGNETIC STRUCTURE DEFINITIONS }

{ Two Triangular structures shifted from 60 degrees }

MOMENT('M1', 'FE11') = FEFRM,          M1,           0; (* M1 // a *)

MOMENT('M2', 'FE12') = FEFRM, M1*COS(120), M1*SIN(120); (* M2 // b *)

MOMENT('M3', 'FE13') = FEFRM, M1*COS(240), M1*SIN(240); (* M3 // -a -b *)

MOMENT('M4', 'FE21') = FEFRM,  M2*COS(60),  M2*SIN(60); (* M4 // a + b *)

MOMENT('M5', 'FE22') = FEFRM, M2*COS(180), M2*SIN(180); (* M5 // -a *)

MOMENT('M6', 'FE23') = FEFRM, M2*COS(300), M2*SIN(300): (* M6 // -b *)

{ DATA COLLECTION }

INTRODUCTION TO MXD.                                                    Page I-8

DATA('T=20K', SCALE, WE=1/SG, MUL=2) IH, IK, IL, IS=-1, RAY, SG;

INCLUDE 'FE2AS.DAT'; { The data are located in this included file }

(* The data format is One line per reflection as this :

h1  k1  l1     { first hkl of a reflection set }

h2  k2  l2     { second hkl of the same reflection set }

..........

hn  kn  ln  1  Int  sigma  { the last hkl following by 1 (Is field)

and in the order Intensity of the set

and related sigma (corrected of Lorentz)}

2   3  -5   1   32.4  1.2  { is an example of single reflection }

END;

I.4  TEMPLATE OF FITTING WITH HETEROGENEOUS DATA (X-RAY AND NEUTRON).

{ X - Ray and neutron mixed fitting data template }

...

{ Get the X-Ray form factor table }

INCLUDE 'FE.FRX'; { we define elemFRX (the form factor) }

INCLUDE 'OX.FRX';

...

{ Get the used magnetic form factor }

INCLUDE 'FE.FRM';

{ For the anomalous diffusion }

{ Define the elemDFPs (delta F prime) and elemDFS (delta F second) }

PARAM

FEDFP = 0.301, FEDFS =  0.845,

OXDFP = 0.047, OXDFS =  0.006;

...

{ Define the Fermi length used (neutron) in E-12  Cm unit }

PARAM

FEb = 0.95,

OXb = 0.58;

{ Define the convention : SELNB = 0 for X-Ray and SELNB = 1 for neutrons }

{  SELNB = 0            1            }

MAGNETIC  NO { X-Ray }, YES{ neutron };

{ Define the general diffusion factors }

PARAM

RFEDIF = SELECT(FEFRX+ FEDFP, FEB), IFEDIF = SELECT(FEDFS),

RFEDIF = SELECT(FEFRX+ FEDFP, FEB), IFEDIF = SELECT(FEDFS);

..

{       ...    possible symmetry definitions     ...      }

...

{ List Of Atoms }

INTRODUCTION TO MXD.                                                    Page I-9

{ Nota all magnetic atoms must be described in the P1 space group }

...

ATOM('Fe34g')  = RFEDIF, IFEDIF, ... ;

...

{ List of magnetic moments }

...

MOMENT('Fe3', 'Fe34g')  = FEFRM, ... ;

...

...

...

{ DATA collection for X-Ray experiment }

DATA('X-RAY',SCALEX, SELNB = 0 {Set for X-Ray }, ... ) ... ;

...

END;

{ DATA collection for neutron experiment }

DATA('Neutron',SCALEN, SELNB = 1 {Set for neutron }, ... ) ... ;

...

END;

I.5  TEMPLATE FOR TWIN OR MAGNETIC DOMAIN MANAGEMENT.

Case of Three (3) twin individuals.

...

...

{ Twin population variables }

VARIABLE PA = 30, PB = 40 { Angle Variable };

LIMITS 1, 89, PA, PB;     { Limited to first sector }

...

{ Twin Population definitions }

PARAM

P1 =   COS(PA)2,  P1 =  SIN(PA)2 ; {  Is same operator than ** }

P2 =  P1*COS(PB)2,  P3 =  P1*SIN(PB)2;

...

{ Use in the data statement }

{ The scale is modified by the

DATA('Twinxxx',(SCALE {Scale factor},

SELECT(P1,P2,P3) { $FN2 correction factor},

SELECT(P1,P2,P3) { $FM2 correction factor),

... )  IH, IK, IL, SELNB, RAY, Sg, IS = -1;

...

(* The data are organized as this (for each measured reflection) :

{ for a peak containing the contribution of

three (3) individuals reflection }

h2  k2  l2   2

h1  k1  l1   1

h0  k0  l0   0    Intensity sigma 1

{ for two (2) individual reflection }

INTRODUCTION TO MXD.                                                   Page I-10

h2  k2  l2   2

h1  k1  l1   1    Intensity sigma 1

{ for single reflection }

h0  k0  l0   0    Intensity sigma 1

*)

END;

The principle is to describe the  structure  for  the  twin  individual

number  0.  If the symmetry operator op transforms the twin individual  n in

the twin individual  0, then the [h0,k0,l0] reflection of the sample  has  a

part   due   to   the   twin   individual    n   that   is   the  reflection

||op||(-1)*[h0,k0,l0].  The contribution of  each  twin  individual  can  be

computed  as  a contribution of the model (reference system of individual 0)

with the appropriated individual weight.

I.6  HELIMAGNETIC STRUCTURE MANAGEMENT.

A/ With one wave vector :

...

WAVEVECT('Q1') = qx, qy, qz; { In the reciprocal lattice coordinates }

...

MOMENT('MK','ATMK','Q1') = MKFRM, RMX, RMY, RMZ, IMX, IMY, IMZ;

...

...

DATA( ... ) IH, IK, IL, NQ, ... ;

WAVEVECT('Q1');

INCLUDE 'datafile'; { in data h = H + nq*q1 }

END;

B/ With two wave vectors.

...

WAVEVECT('Q1') = q1x, q1y, q1z ;

WAVEVECT('Q2') = q2x, q2y, q2z ;

...

{ Atoms }

ATOM('xx') = ... ;

...

{ Moments }

MOMENT('Q1Mxx','xx','Q1') = ... ;

MOMENT('Q2Mxx','xx','Q2') = ... ;

...

…

The data file model is :

{ IH  IK  IL  NQ                    IS=-1      RAY     SG }

WAVEVECT('Q1');{ the Wave vector is Q1 for the next reflection }

INTRODUCTION TO MXD.                                                   Page I-11

{  a set with one nuclear and two Q1 satellite reflections }

3   0   0    0   { Nuclear   H }

3   0   0    1   { Satellite H+q1 }

3   0   0   -1   { Satellite H-q1 } 1        32.5    0.8

{  a single Q2 satellite reflection }

WAVEVECT('Q2');{ the Wave vector is Q2 for the next reflection }

3   1   0   -1   { satellite H-q2 } 1        41.6    1.2

{  a set of one Q2 satellite and one Q1 satellite reflection(s) }

2   3   1   -1   { satellite H-q2 }

WAVEVECT('Q1');{ the Wave vector is Q1 for the next reflection(s) }

2   2   2    1   { satellite H+q1 } 1        27.1    2.4

...

C/ Model description of a helimagnetic structure with the use of HELMOM

macro statement.

HELMOM is a name of a user defined macro that can be used  to  describe

all  helimagnetic structures.  The use of the HELMOM macro is illustrated in

the next template.

...

MACROCALL HELMOM; { Load the HELMOM macro from the standard MXD library }

...

...

HELMOM 'Q1Mxx', 'xx', 'Q1', xxFRM, xxOM1, xxCH1, xxPH1, xxEU1, xxEV1, xxAL1;

HELMOM 'Q2Mxx', 'xx', 'Q2', xxFRM, xxOM2, xxCH2, xxPH2, xxEU2, xxEV2, xxAL2;

...

...

The use of the HELMOM macro is very  convenient  for  the  helimagnetic

structures.   OMi,  CHi  and  PHi  are  the  eulerian angles that define the

orientation of an ellipse with the main axis EUi and EVi.  When the Eulerian

angles  are zero Eu is along the X axis (of working reference system) an EVi

along of Y one's.  ALi is the circulation  phasis  along  the  ellipse  that

define the relative phasis of each modulated moment.  FRM is the appropriate

form factor to use.  The three  first  parameters  are:   the  name  of  the

moment, the name of attached atom, and the name of the related wave vector.

I.7  BECKER AND COPPENS EXTINCTION MANAGEMENT.

In MXD the Becker and Coppens extinction is very easy to use as can  be

seen below.

{ In first we can attach the Becker and Coppens extinction library }

MACROLIB 'MXDLIB:EXTINC.MXL';{attach the Becker and Coppens ext. macro lib.

}

...

{ We must define the used wavelength }

INTRODUCTION TO MXD.                                                   Page I-12

PARAM $LAMBDA = 0.89 {Wavelength in angstroem };

...

{ Define a data field for the average way in the crystal computed from

a previously running absorption program }

{ If the crystal is spherical, we can use an "INCLUDE 'ABSTAB.MXD' to define

a pseudo form factor with the absorption and average way tables ( created

by the execution of the VAX/VMS/DCL command MXD MXDLIB:ABSORB ) }

FFASSIGN E$TMU:-1; { Allocate a place at the end of form factor table }

...

{ call the extinction macro }

EX$BECCOP 0 {neutron}, 0 {isotropic}, 1{crystal type 1},

1 {Lorentzian distribution };

{ EX$BECCOP has defined the symbol E$Y has the current extinction

coefficient }

...

...

...

{ The use in the data statement is : }

DATA('name',(Scale, E$Y), ... ) ...;

...

END;

{ or for previously reduced data collection }

DATA('name') = Scale, E$Y;

I.8  POLARIZED NEUTRONS MANAGEMENT.

MXD gives the full support for flipping ratio and polarization  analyse

experiments.   In the next example we give a template data file to execute a

least-squares on a not centered structure ( Space group  P-62m  )  with  the

coumpound MnRuAS ( B.Chenevier thesys).

TITLE ' Mn Ru As polarized neutrons (I.L.L. D3), Magnetic field //(010).';

{ account for the lambda/2 contribution }

{  SELNB = 0  for lambda and 1 for lambda/2 }

PARAM LAMBDA = 0.924,

$LAMBDA = SELECT( LAMBDA, LAMBDA/2 );

INCLUDE 'ABSTAB.MXD';         { include the absorption definitions

of $ABS and E$TMU }

FFASSIGN $ABS:-1, E$TMU:-2    { assign the two last column in the

binary data files }

PARAM $F2 = $F2POLA;          { define the base to compute the

extinction coefficient E$Y }

{ Use the Becker and Coppens Extinction correction }

MACROLIB 'MXDLIB:EXTINC.MXL'; { Enable the use of extinction library }

MACROCALL EX$BECCOP;          { load the macro }

INTRODUCTION TO MXD.                                                   Page I-13

{ call the extinction macro }

EX$BECCOP 0 {neutron}, 0 {isotropic}, 1{crystal type 1},

1 {Lorentzian distribution };

...

...

...

{ Creates the file "DATA.MK3" for the FOURPL program of MK3 library }

OPTION(MK3OUT)= YES, 0,0,1;

...

...

...

{ Unit Cell }

CELL 6.5212,,3.6061,,,120;

{ space group }

LATTICE 'P';                     { Specify the lattice }

SYMTRY('-6')  =  0, -1,  0,   0, { specify the -6 axis }

1, -1,  0,   0,

0,  0, -1,   0;

SYMTRY('MU')  =  0,  1,  0,   0, { specify the Mu mirror }

1,  0,  0,   0,

0,  0,  1,   0;

GENSPACE;                        { generate all the space group }

...

...

...

{ definitions of atoms variables  }

VARIABLE XMN   =-0.419, XRU   = 0.254;

VARIABLE URU = 0.002, UMN = 0.004, UAS = 0.001;

...

...

...

{ definion of magnetic moment components }

VARIABLE  MN1 = 3.03, MN2 = 3.79;

...

...

...

{ Form Factor with references }

(* FACTEUR DE FORME DE MN  - SOURCE MNSB *)

(* FACTEUR DE FORME DE RU  - SOURCE FERH (SHIRANE ET AL., PHYS.REV (1964))-

*)

PARAM MNFRM(0,0.02) = (1,

0.995, 0.985, 0.970, 0.945, 0.915, 0.875, 0.835, 0.785, 0.735, 0.680,

0.635, 0.585, 0.535, 0.490, 0.450, 0.410, 0.375, 0.345, 0.315, 0.285,

0.255, 0.230, 0.210, 0.190, 0.180);

INTRODUCTION TO MXD.                                                   Page I-14

...

...

...

{ Fermi Lengths - Source : International Tables }

PARAM BMN = -0.373,

BRU =  0.721,

BAS =  0.658;

...

...

...

{ Crystallographic structure }

{ All the magnetic sites must be described }

ATOM('MN11') = BMN,,    1,  XMN,    0,  1/2,  UMN;

ATOM('MN12') = BMN,,    1,    0,  XMN,  1/2,  UMN;

ATOM('MN13') = BMN,,    1, -XMN, -XMN,  1/2,  UMN;

{ For the not magnetic atoms we use the space group information }

ATOM('RU'   ) = BRU,,  1/4,  XRU,    0,    0,  URU;

ATOM('AS1' ) = BAS,, 1/12,    0,    0,  1/2,  UAS;

ATOM('AS2' ) = BAS,,  1/6,  1/3,  2/3,    0,  UAS;

...

...

...

{ Magnetic structure definitions  }

MOMENT('MN11','MN11') = MNFRM, -MN1;

MOMENT('MN12','MN12') = MNFRM,  MN2;

MOMENT('MN13','MN13') = MNFRM,  MN2;

...

...

...

{ Compute (with sigma) the magnetization in Bohr magneton per formula unit }

PARAM MRES = (MN1 + 2*MN2)/3;

...

...

...

{ Data collection definitions }

{ Proportion of LAMBDA / 2 intensity variable definitions }

VARIABLE TOLS2 = 0.004;

LIMITS TOLS2,0,1; { limited to the range 0..1 }

UNFIXED TOLS2;    { must be refined }

{ Definitions of the efficiences of the polarization

for each wavelength }

VARIABLE EFF=0.9, EFFL2 = 0.30;

LIMITS 0,1, EFF, EFFL2;

UNFIXED EFF; UNFIXED EFFL2;

{ Magnetic field on the sample }

PARAM HAPPL = 4.5 {TESLAS};

INTRODUCTION TO MXD.                                                   Page I-15

PARAM EFFE = SELECT( EFF, EFFL2 );  { define the polarization states }

{ for each wavelength }

{ define the two used polarization directions }

NPOLADIR('100') = HAPPL, 0, 0,      { Magnetic field components }

EFFE, EFFE,       { Spin up and down pol. efficiences }

E$Y               { secondary extinction correction }

*SELECT( 1, TOLS2); { with LAMBDA/2 effect }

DATA('D3EXP',1, RAY=R, NPOLA=3{+/-}, WE=1/SG, SELNB = IS < 0 )

{ Field to read on each line of data }

IH, IK, IL, IS=-1, R, SG;

NPOLADIR('100');

(*

{ each measured reflection in the data file has two lines }

2   4   2                      { the lambda/2 reflection }

1   2   1   1    3.5275 0.0022 { the lambda reflection }

*)

INCLUDE 'MRUA010POL.HKLSLS2'; { get the data file }

END;

It is possible to handle many polarization direction data together.

In this last case, the user must describe the magnetic moments components

in a true arrangement for each direction of the applied magnetic field.

It can take advantage of the predefined symbols $HX, $HY, $HZ that denote

the current components of the magnetic field on the sample in the working

space.

CHAPTER II

THEORETICAL MXD BACKGROUND.

In this section we describe the theoretical background of the MXD least

square  fitting  process  with all used related formulae.  The used software

technics are derived from the list  management  methods  that  the  ordinary

physicist users do not known.

II.1  ATOMIC STRUCTURE FACTOR SUMMATION CONVENTION.

MXD is written to handle all very complicated magnetic and/or modulated

structures, so a general symmetry operator convention is taken :

For any magnetic or modulated (in position or in statistic  population)

atom,  MXD  applies  the P1 space group.  The space group information is not

used  when  the  atom  carries  a  magnetic  moment,  or  has  an   attached

crystallographic  modulation, the user must describe all equivalent atoms in

these structures.  The consequence is to give a particular name to each atom

in the crystallographic site that is very convenient to describe any form of

constraints on the modulations and/or magnetic moments.

For the normal atoms (not magnetic and not  modulated)  the  user  must

give  a modified population with the appropriate site multiplicity ( see the

first and second examples of the first chapter "INTRODUCTION TO MXD").

In all, other paragraphe of this chapter the next convention is used to

fit with any printer restrictions :

The summation operator below :

final_value

______________________

\

\                      ...EXPRESSION...

/

/_____________________

index = initial, step

THEORETICAL MXD BACKGROUND.                                            Page II-2

is denoted as this :

finalvalue

SUMM                      ...EXPRESSION...

index = initial, step

and the step will be omitted if it is 1.

An other convention is to use pi to denotes the pi number 3.1415927...

II.2  ATOMIC STRUCTURE FACTOR.

The structure factor is computed with the formula :

->    all_atoms              ->             ->  ->

Fn(h) = SUMM         { PP * fn (h)*EXP(2*pi*i* H * r )

j = 1           j    j                     j

2    ->         ->

* EXP(2*pi * < h ! beta ! h >) }

j

i is the square root of -1.

with :

j     atomic number,

r     atomic position vector,

PP    atomic statistic occupancy factor (population),

h     true scattering vector,

H     attached reciprocal lattice vector,

fn    atomic (or nuclear for neutron) scattering factor,

beta  Thermal tensor.

and :

->  ->       ->

h = H + nq * q,

beta   = U   / (b * b )

ij    ij     i   j

THEORETICAL MXD BACKGROUND.                                            Page II-3

where :

q is the current wave vector, and nq an integer.

U is the quadratic thermal vibration tensor and

*   *     *

b    are the unit cell period vectors A , B and C .

i

II.3  MAGNETIC STRUCTURE FACTOR.

The magnetic structure factor Fm has this expression :

--> ->    all_moments     -->      ->             ->  ->

Fm (h) = SUMM             MM * fm (h)*EXP(2*pi*i* H * r )

j = 1             j    j                     j

The effective magnetic structure factor FM

which gives the diffraction is :

-->       -->   -->  -->    -->

FM   =  [ hn * (Fm * hn ) - Fm ] * (e * gamma)/(m * c )

Where :

fm    is magnetic form factor,

MM    is the complex magnetic moment vector,

hn    is an unitary vector parallel to the h scattering vector,

e     is the electron electric charge,

gamma is the neutron giro magnetic factor,

m     is the neutron heavy and

c     is the light speed.

The complex magnetic moment is equal to the real magnetic moment if the

computed  reflection  has  h  =  H  (no satellite reflection) or if the wave

vector is flagged to be rational.

If it is a satellite reflection of a helimagnetic structure,  then  the

nq number is +1 or -1 and we can define :

->   ->  ->           ->   ->  ->

h+ = H + q    and     h- = H - q.

For the h+ satellite the MM vector is the adjoint of M  complex  vector

and  for h- one's MM and M vector are the same.  The M complex vector can be

written as :

THEORETICAL MXD BACKGROUND.                                            Page II-4

->                    ->                  ->                  ->

M  =  (RMX + i*IMX) * x + (RMY + i*IMY) * y + (RMZ + i*IMZ) * z ;

RMX, IMX, RMY, IMY, RMZ and IMZ are the real and imaginary part of

the complex vector M used to describe any modulated magnetic structure.

The physical magnetic moment at the r coordinate is given as :

->    ->               ->  ->    ->*               ->  ->

m  =  M  * EXP(2*pi*i* q * r ) + M  * EXP(-2*pi*i* q * r )

->*                                   ->

With M   denotes the complex adjoint of the M  vector.

The x, y and z are the unitary vector of a work space defined from  the

unit cell by the MXD CELL or RCELL statements.

The more current work space is :

->   -> ->             -> ->    ->                      ->   ->

x // a, y in the plane a, b and z the direct product of x by y.

II.4  POLARIZED NEUTRON INTENSITIES.

In the polarized neutron case the total intensity is deduced  from  the

four  partial intensities I++, I--, I+- and I-+.  If we define the reference

system u, v, w where w is direction of the polarization  magnetic  field  on

the sample, then we can write :

2

I++  =  | Fn + FMw |  / 2,

2

I--  =  | Fn - FMw |  / 2,

2

I+-  =  | FMu + i * FMv |  / 2,

2

I-+  =  | FMu - i * FMv |  / 2.

In these formulae the FMu, FMv  and  FMw  are  the  components  of  the

magnetic structure factor vector FM and Fn is the atomic (nuclear) structure

factor.  The formulae (3) and (4) are not very computing convenient then MXD

use following form instead :

THEORETICAL MXD BACKGROUND.                                            Page II-5

->  -->  -->

I+-  = |  FMuv   + ( hn, RFM, IFM ) | / 2,

2      ->  -->  -->

I-+  = |  FMuv   - ( hn, RFM, IFM ) | / 2,

where :

->         ->

FMuv = | FMu * u  + FMv * v |.

-> -> ->                                            -> ->    ->

( a, b, c ) denotes the mixed product of three vectors a, b and c.

II.5  MODULATED ATOMIC STRUCTURE MANAGEMENT.

The atomic (nuclear for neutron) modulation management of MXD is  based

on  the  normal  3  Dimension  space  use but it is possible to use an other

approach by the use of appropriate user MXD macros.   The  basic  modulation

description  use the complex displacement vector U and the complex occupancy

probability factor P are defined :

->                    ->                  ->                  ->

U  =  (RUX + i*IUX) * x + (RUY + i*IUY) * y + (RUZ + i*IUZ) * z ;

with the real and imaginary components RMX, RMY, RMZ and IMX, IMY, IMZ.

P  =  Pmd * EXP(i*Pph)

The physical displacement and population at the r coordinate are given

as :

->    ->              ->  ->    ->*               ->  ->

u  =  U * EXP(2*pi*i* q * r ) + U  * EXP(-2*pi*i* q * r )

->  ->     *                ->  ->

p  =  P * EXP(2*pi*i* q * r ) + P  * EXP(-2*pi*i* q * r )

->*                                   ->

With U   denotes the complex adjoint of the U  vector

*

and  P   the complex adjoint of P scalar.

For a modulated atomic population the atomic structure factor is  given

by :

THEORETICAL MXD BACKGROUND.                                            Page II-6

->             ->  ->  ->

for h+ reflection ( h = H + q ) :

-->    all_atoms      *      ->             ->  ->

Fn(h+) = SUMM         { P * fn (h)*EXP(2*pi*i* H * r ) * <thermal

factor>}

j = 1          j    j                     j

->             ->  ->  ->

for h- reflection ( h = H - q ) :

-->    all_atoms             ->             ->  ->

Fn(h-) = SUMM         { P * fn (h)*EXP(2*pi*i* H * r ) * <thermal

factor>}

j = 1          j    j                     j

These formulae have the same form as their equivalent for  helimagnetic

structure.   but  in  the  case  of  a  unique  q wave vector in a modulated

displacement and occupancy probability the formulae are :

->     ->     all_atoms            ->             ->  ->         ->

Fn(H + nq*q)  = SUMM       { PP * fn (h)*EXP(2*pi*i* H * r ) * g    (h) }

j = 1         j    j                     j     nq,j

With :

->        ->              ->               ->

g   (h) = G   (h) + P * G     (h) + P  * G     (h),

n,j       n,j       j   n-1,j       j    n+1,j

->

G   (h) = EXP(i*[-n*phi + |n|*pi/2]) * J   (2*rho ),

n,j                   j                |n|      j

->  ->

rho * EXP(i*phi ) = 2*pi* h * U ,

j           j               j

and J  denotes the n'th first kind Bessel function.

n

The G function is used only in the unique q wave vector  case;  in  all

other  cases  the user must use the DISCRET atomic model to correctly handle

the multi wave vectors structures.

II.6  CALCULATED AND OBSERVED VALUES.

The value compared for a given  observation  is  defined  by  the  DATA

THEORETICAL MXD BACKGROUND.                                            Page II-7

statement for each Data collection.

In all cases the Scale factor is applied to the final calculated  value

found  to  compare  it to the given observation.  Therefore, scale factor is

always find with one (1) least square cycle only.

You have four (4) kinds of Data Collection :

1.  Observed intensity data (RAY).

2.  Observed squared structure factor (F2).

3.  Observed structure factor (SF).

4.  Polarized neutron diffraction data.

II.6.1  Intensities.

The used formula is :

all [hkl] in sets                    2          --> 2

C  =  Scale * SUMM               mul * ( Cn * | Fn |  + Cm * | FM |  )

l                    l      l      l       l      l

Where mul is the multiplicity of the l'th reflection,

Cn and Cm the related extinction correction factors (default 1),

and scale the current scale factor.

II.6.2  F2 Observation Data.

2            --> 2

C  =  Scale * ( Cn * | Fn |  + Cm * | FM |  )

Where Cn and Cm the related extinction correction factor (default 1),

Scale the current scale factor.

THEORETICAL MXD BACKGROUND.                                            Page II-8

II.6.3  Structure Factor Based Observation Data.

2            --> 2  1/2

C  =  Scale * ( Cn * | Fn |  + Cm * | FM |  )

Where Cn and Cm the related extinction correction factors (default 1),

Scale the current scale factor.

II.6.4  Polarized Neutron Case.

It is a special case.  In fact, you can define a neutron polarized data

collection  as  based  on SF, F2 or RAY (Intensity).  But in general the RAY

mode is more convenient.  If you have a flipping ratio data collection,  the

scale factor must be fixed to 1.

You can use the different formulae as below :

If you have a polarizer and an analyser on your goniometer :

IC = Cp++ * I++,                 for ++ measured reflection,

IC = Cp-- * I--,                 for -- measured reflection,

IC = Cp+- * I+-,                 for +- measured reflection,

IC = Cp-+ * I-+,                 for -+ measured reflection,

If you have not an analyser on your goniometer :

IC = IC+ =  Cp++ * I++ + Cp+- * I+-, for + measured reflection,

IC = IC- =  Cp-- * I-- + Cp-+ * I-+, for - measured reflection,

IC = IC+ / IC- for flipping ratio in F2 or Sf data collection and

all ref.            all ref

C = Scale*{ SUMM mul * IC+ } / { SUMM mul * IC- }

l      l    l        l      l    l

for flipping ration in Ray data collection.

Where the Cpxx factor are  the  related  correction  factors  including

flipper efficience factor and/or extinction coefficient.

Except for the last case, C is given by the next formulae :

THEORETICAL MXD BACKGROUND.                                            Page II-9

all ref.

C = Scale * SUMM    {  mul * ICxx }   for RAY based data collection,

l            l      l

Where ICxx is any IC intensity (xx = +, -, ++, --, +-, -+, +/-).

C = Scale * Icxx     for F2 based data collection,

and

1/2

C = Scale * | Icxx |     for SF based data collection.

II.7  LEAST SQUARE FORMULAE.

Now we have defined the computed quantity C for each  p'th  observation

value O.  Each observation has related weight We and sigma Sig.

The total number of reflections (across all defined  data  collections)

is  noted  Nobs  and the total number of fitted variable is Nvar (across all

defined diagonal blocks).

If the i'th fitted variable are denoted  qi,  then  we  have  the  next

definitions :

With the standard weight given as We = 1 / Sig .

Nobs                   2

SUMM  { (O  - C ) * We }

2            p        p    p      p

CHI    =   --------------------------------

Nobs - Nvar

1/2

(      Nobs                   2   )

(     SUMM  { (O  - C ) * We }    )       The weighted

(      p        p    p      p     )

R      = ( ------------------------------- )    Least Square R Factor

w.lsq   (        Nobs             2       )

(       SUMM  { O  *  We }        )      "WEIGHTED LSQ R"

(        p       p      p         )

THEORETICAL MXD BACKGROUND.                                           Page II-10

The crystallographic weighted R factor is given by th formula :

Nobs

SUMM  { |O  - C | * We }            The weighted

p        p    p      p

R      =   -------------------------------      Crystallographic R Factor

w.abs            Nobs

SUMM  | O  *  We |               "WEIGHTED ABS R"

p       p      p

The unweighted versions of these R factors are deduced from these  ones

by the change of We to 1.

The least square process consists ( for each diagonal block) in :

1/  Building the matrix coefficients :

d C       d C

Nobs       p         p       2

M     = SUMM   ------- * -------- * We ,

i,j     p       d Q       d Q        p

i         j

d C

Nobs                             p

B     = SUMM   ( O   -  C  ) * We  * -------

i       p        p      p       p     d Q

i

2/  Solving the linear system :

|| M || * || delta_q || = || B ||,

-1

with the matrix definition || N || = || M ||  ,

the system becomes :

|| delta_q || = || N || * || B || .

3/  Applying the variable shift delta_q to the q variable :

Q   =   Q   + DMP * delta_q

i       i                 i

where DMP is the dampening factor.

THEORETICAL MXD BACKGROUND.                                           Page II-11

4/ Computing the variable and related function sigma as this :

The sigma of each variable :

2  1/2

sig    = [ N    * CHI  ]   ,

i        i,i

The correlation cofficients are computed as :

(         2      ) 1/2

(       N        )

(        i,j     )

C      = (----------------)

i,j     (   N    * N     )

(    i,i    j,j  )

The function dependant (MXD Variable Parameter) :

f(Qi1,Qi2...Qin)    have the sigma given as :

2     Nvar Nvar         d f      d f

sigma(f(Q1,..Qn)  =  SUMM SUMM  C    * ------ * ------ * Sig  * Sig  .

i    j     i,j    d Q      d Q        i      j

i        j

CHAPTER III

SYNTAX ELEMENTS OF THE MXD LANGUAGE.

The description of the syntax of the various  statement  primitives  of

MXD use some common notations and rules that are defined below.

III.1  KEYWORD NOTATION.

MXD statements are based on the use of reserved words (so called  keywords),

which  cannot  be  used for any other purpose.  All the keyword are noted in

CAPITAL letters in this manual, but,  in  a  real  data  file  MXD  proceeds

correctly  with  minor  characters  because,  except in the string character

input MXD translate in capital all input letter.

CENTER tells MXD to operate  in  centered  structure  mode  (centered  space

group).  Our notation is CENTER but center is also legal.

III.2  PRIMITIVE PARAMETER NOTATION.

The words surrounded by the bra-ket characters "<" and ">" denote a symbolic

form.  We use some symbolic notations :

III.2.1  Numeric Expression Notation.

<c#n> denotes any immediatly evaluable numeric expression

and <#n> denotes any numeric expression which is not necessarily  immediatly

evaluable  because  some  special  parameter or least-square variable can be

referenced in this expression.

The Unit Cell definition and Atome primitives give a  good  example  of  the

difference:

SYNTAX ELEMENTS OF THE MXD LANGUAGE.                                  Page III-2

CELL <c#a>, <c#b>, <c#c>, <c#alpha>, <c#beta>, <c#gamma>;

Is the precise syntax of the Cell primitive.  The unit cell  parameters  can

be the result of some computing but they are fixed.

ATOME('Fe23')= <#rdif>, <#idif>, 1,

<#xfe23>, <#yfe23>, <#zfe23>, <#ufe23>;

This primitive creates an atom with the name Fe23,  the  complex  scattering

factor  <#rdif>,<#idif>, the coordinates <#xfe23>,<#yfe23> and <#zfe23>, and

the  isotropic  thermal  factor  <#ufe23>.   All  atom  parameters  can   be

variable-dependant.

III.2.2  String Constant Expression Notation.

<s#name> denotes a string expression parameter .

Example:

TITLE <s#title_string> ; define the given string as the title of the present

work.

<l#val> denotes a character specification parameter.

A character specification can be a single character or a  string  where

the first character is used as this :

Example:

LATTICE <l#lattice> ; { can be used }

LATTICE R; {,} LATTICE 'RHOMBOEDRIC'; { and } LATTICE 'r'; {or}

LATTICE 'R';

Give the same information :  The crystal lattice is a Rhombohedral lattice.

III.2.3  Identifier Notation.

<ident> denotes an legal identifier name.

An identifier name is a string of alphanumeric character where the first one

is  an  alphabetic character.  An identifier name can have any size but only

the first eight character are used, then,  two  identifiers  with  the  same

first eight characters are not distinguished.

The alphabetic characters are:

the letters A To Z, $, the underline character "_" and #.

The alphanumeric characters are the digits, and the alphabetic characters.

SYNTAX ELEMENTS OF THE MXD LANGUAGE.                                  Page III-3

III.2.4  Expression Operators.

The list of more usual operators and standard function is given below:

<#e1>+<#e2>          addition operator.

<#e1>-<#e2>          substract operator.

<#e1>*<#e2>          multiply operator.

<#e1>/<#e2>          divide operator.

<#e1>^<#e2>

or <#e1>**<#e2>         exponential operator.

-<#e>            unary minus operator( sign change

operator).

+<#e>            unary plus operator(no effect).

SIN(<#e>)          sine.

COS(<#e>)          cosine.

TAN(<#e>)          tangent.

LOG(<#e>)          natural logarithm.

EXP(<#e>)          exponential.

ASIN(<#e>)          arcsine.

ACOS(<#e>)          arccosine.

ATAN(<#e>)          arctangent.

TANH(<#e>)          hyperbolic tangent.

SQRT(<#e>)          square-root.

ATAN(<#eb>,<#ea>)        Phasis of a + i*b.

III.3  INPUT DATA FORM.

The MXD Input  Format  Is  Free.   The  Primitive  name  with  this  related

parameters  (so  called  Statement)  can  be  split  in  many line, and many

statements can be written on the same input line.

Each statement is ended by a semicolon character.  This is  a  general  rule

for any MXD statement.

The first part and/or the Last part of the input line can be ignored by  MXD

by the use of the F option of the PRAGMA primitive.

III.4  OPTIONAL AND OBLIGED PRIMITIVE PARAMETERS.

Many primitives can have a  variable  number  of  parameters.   When  a

parameter  is optional the parameter is surrounded by the two characters "["

and "]".  Then the exact syntax of the CELL primitive use is:

CELL <c#a> [, [<c#b>] [, [<c#c>] [,

[ <c#alpha> ] [, [<c#beta>] [, [<c#gamma>] ] ] ] ] ] ;

The <c#a> parameter must be present in the CELL primitive,  the  others

are optional.

CELL 4.04;             (* for a cubic unit cell of 4.04 angstroem *)

SYNTAX ELEMENTS OF THE MXD LANGUAGE.                                  Page III-4

CELL 8.31,,12.43,,,120;(* for an hexagonal unit cell with a=8.31,c=12.43 *)

In all the next statement syntax description the notation is  simplified  by

the suppression of the all internal bra-ket characters as below :

CELL <c#a> [,<c#b>, <c#c>, <c#alpha>, <c#beta>, <c#gamma> ];

III.5  ALTERNATIVE CHOICE NOTATION.

The source listing of the current MXD data file can be set off or on by

the two MXD statements :

PRAGMA L-;  { Set listing off }

...         { this part of the MXD data must not be showed in the listing }

PRAGMA L+;  { set listing on }

The user must define a choice "+" or "-".  So  our  notation  for  this

form of this statement is :

PRAGMA L {+|-};

The "|" character separates each possibility of choice and "{" and  "}"

indicate the begining and the end of the choice list.

III.6  GENERAL MXD STATEMENT FORM.

We describe below the use  of  each  physical  MXD  primitive  to  form

correct MXD statements.

There are Three types of MXD statements :

The  PRIMITIVE  statements,  the  ASSIGNATION   statements,   and   the

STRUCTURED (for special use) statement.

A PRIMITIVE STATEMENT is begun  with  a  primitive  keyword  with  this

related   parameters,   if   necessary,   and   is   ended  by  a  semicolon

character(";").

An ASSIGNATION STATEMENT has the general form :

<ident> = <expression> ;

The expression must match with the identifier type.

SYNTAX ELEMENTS OF THE MXD LANGUAGE.                                  Page III-5

This statement is used to assign a new value to a constant parameter  or  to

change  an  initial  value  of  a  variable, this is specially used to set a

variable at this last refinement value.

First, we give the legal syntax of the  statement  and  after,  all  related

explainations or particularities.

CHAPTER IV

DATA MANAGEMENT PRIMITIVES OF MXD.

The MXD data  file  organization  is  the  technique  to  have  a  well

structured  and  easy to understand data structure.  The MXD data management

primitives help the user to write well suited data file.

IV.1  HEADING STATEMENT.

You can give a title to a MXD job by use of the statement:

TITLE <s#title> ;

The character string <s#title> is printed on the  first  line  of  all  next

listing page.

Example:

TITLE ' Raffinement of U_Alpha room temperature structure ';

IV.2  THE PRAGMA STATEMENT.

If you want to modify  some  working  mode(s)  the  MXD  data  compiler

(MXDCMP), you can use the statement PRAGMA, the syntax is given below :

PRAGMA <opt1> [, <opt2>, <opt3>, ... ] ;

Each given options <optn> has the form :

<l#letter> and appropriate parameter(s) must be an element of  th  list

below :

1.  L[{+|-}][:n] To Enable/Disable the MXD source listing.

DATA MANAGEMENT PRIMITIVES OF MXD.                                     Page IV-2

2.  D{+|-} To Enable/Disable the DATA interpretation listing.

3.  M{+|-} To Enable/Disable the MACRO expansion listing.

4.  F:nf:nl To set the source data column range nf..nl.

5.  N{+|-} To enable/Disable fatal error on not existing file.

IV.2.1  The L Option For List On Or Off.

The L option commands the MXD source listing output.

+ enables the listing output and - disables it.

n if given, changes the current INCLUDE level to disable the output.

The default is L-:1 at the MXDCMP start time, but in the normal use  of

the  MLS command (in VAX/VMS systems) this default is changed to L+:1 by the

statement "PRAGMA L+:1;" in the MXD.INI file used  to  initialize  the  data

compiler.

The include level for source listing n is the  nested  level  limit  to

really output the source listing.

Example :

{ Basic file a.mxd            file b.mxd               file c.mxd }

{ nested level = 1 }

INCLUDE 'b.mxd'; -------> { nested level = 2 }

{ no listing because

the default is L+:1 }

...

INCLUDE 'c.mxd';---------> { nested level = 3 }

{ listing

off the first call

and

on the second call}

...

...

{ listing enabled } <----  ENDFILE;

PRAGMA L:2;

{ listing on for this file not for c.mxd }

INCLUDE 'c.mxd';

...

...

PRAGMA L:3;

{ the first part of the

file c.mxd will be

output on the listing }

INCLUDE 'c.mxd';

...

DATA MANAGEMENT PRIMITIVES OF MXD.                                     Page IV-3

{ all next        <------- ENDFILE;

included file

will be output on

the listing }

...

IV.2.2  The D Option For Data Interpretation Listing.

The D+ enables the DATA interpretation listing and D- disables it.   If

it  is  enabled  then the DATA statement prints each interpreted reflection.

It can be useful in some complicated data reduction process.

The default is D-.

IV.2.3  The M Option For Macro Expanssion Listing.

The M{+|-} option  Enables  or  Disables  the  listing  of  the  really

executed  statement during a macro interpretation.  This option must be used

to debug a macro.

The default is M-.

IV.2.4  The N Option For Not Existing File Error Handling.

When MXD tries to open a file  by  the  INCLUDE,  CHAINE,  or  OPENFILE

statement, if the file cannot be opened, the default ( N-) effect is to give

no difference between a non existing file and an empty  existing  file.   If

the  N+  option  is  enabled,  then  the  not  existing  file fatal error is

generated.  It is a good pratique to specify the N+  option  when  the  user

include some important file as space groupe or form-factor definitions.  The

symbol $STATUS can be used to know the success of the INCLUDE  statement  as

this:

INCLUDE 'file.MXD',N-;

IF $STATUS = 0 THEN

{ action when success }

ELSE

{ action when open fails }

ENDIF;

IV.2.5  The F Option For Source Input Format.

The F option is used to ignore some column in the mxd source file ( the

DATA MANAGEMENT PRIMITIVES OF MXD.                                     Page IV-4

user  written mxd statements ).  nf is the first column number and ln is the

last one.  Only the column nf to nl will be read by the MXD compiler.

A good example is :

PRAGMA F:2:72;

We ignore the first character of each line and all characters after the

72'th column that can be used to line sequence number.

The default is PRAGMA F:1:120;

IV.3  INCLUDE AND CHAINE STATEMENT.

If you have a set of mxd data line  that  is  not  particular  to  your

problem, you can use the INCLUDE statement.

The syntax of INCLUDE and CHAINE statement is :

INCLUDE

or     <s#filename> [, <opt1>, <opt2>, ... ];

CHAINE

The INCLUDE statement directs the input stream to the  filename  source

MXD  file  with keep the current source context.  You can use up to four (4)

nested INCLUDE statements in interractive mode and three  (3)  in  automatic

run  mode.   Each  INCLUDE  statement increases by one the include level and

each end of file (or ENDFILE statement) decreases by one the current include

level.   The  current  set of option (see PRAGMA statement) is copied in the

included file context and, modified by the given option(s) <optn> if  given.

When the control is returned to the caller source file (where is the INCLUDE

statement), the previous options are restored.

The CHAINE statement directs the input stream to  the  filename  source

MXD  file  without  keeping  the current source context.  The current set of

option is not saved, it can be modified by the given option(s) <optn>.

IV.4  ENDFILE STATEMENT.

The ENDFILE statement is equivalent to the true end of file.   When  it

is reached, The MXD compiler, as with a true end of file, closes the current

MXD source file to return to the previous context.

The ENDFILE syntax is :

ENDFILE ;

DATA MANAGEMENT PRIMITIVES OF MXD.                                     Page IV-5

IV.5  THE MXD SOURCE INPUT FILE LOGIC.

When MXDCMP (the Data Compiler) is started, during the initialize time,

it looks for a data file with the name MXD.INI .

If this file does not exist, then the terminal mode is enabled and  the

MXDCMP prompt is displayed on the user terminal.

In this case, the default PRAGMA is :

PRAGMA L-:1,D-,M-,F:1:120;

If the MXD.INI file exists, then an automatic INCLUDE to this  file  is

performed.  Below, we show a typical MXD.INI file :

{ auto start comment }

INCLUDE MXDLIB:'MXD.OPT';  { to define all particular symbols }

CHAINE 'userfile.MXD',L+; { to begin the mxd file interpretation }

{ and turn on the source listing }

The MXDCMP program has a  limitation  of  four  (4)  simultaneous  open

source files.  Therefore in interactive mode you can have really four (4) of

nested INCLUDEs, but in the automatic mode (general case) you can  use  only

three (3) nested INCLUDEs because you must take the main source data file in

the four files.

With these statements you can write well efficient data files.

Examples :

If you have many data that use the same space group Ia3d you can create

the file IA3D.MXD as this :

{ SPACE GROUP IA3D }

LATTICE 'I'; CENTER; { set lattice and center at origin }

SYMTRY('3  ') = 0, 1, 0,   0,

0, 0, 1,   0,

1, 0, 0,   0; { matrix of operator (3!   0   0   0)

}

SYMTRY('4.1') = 0,-1, 0, 1/4,

1, 0, 0, 3/4,

0, 0, 1, 1/4; { matrix of operator (4! 1/4 3/4 1/4)

}

GENSPACE;                     { generates the full space group }

{ in the each main mxd source file }

INCLUDE 'IA3D.MXD'; { include the space group definition }

You can also use this method to use a collection of form  factor  files

to have only one version of each.

DATA MANAGEMENT PRIMITIVES OF MXD.                                     Page IV-6

IV.6  THE APPLICATION PROGRAM OPTION SPECIFICATIONS.

To give some directive to the application  program  MXD  as  a  special

statement  OPTION  that  can  be used to transmit some numeric values to the

application program.  The syntax is given as :

OPTION(<c#optionnumber>) =

<c#val1> [, <c#val2> [, ... <c#val8> ] ] ;

A maximum of eight (8) values can be  transmitted  to  the  application

program.   The  standard  options as defined in the MXD.OPT file has integer

constant value and may be used to give more readely MXD data file.

The standard options for MXDLSQ are defined in  the  file  MXD.OPT  (in

MXDLIB:  for VAX/VMS system) with some defined value as this :

PARAM SHORTLST = 0, NCYCLE = 1;

PARAM NO = 0, YES = 1, ALWAYS = 2;

Some option number are reserved as global option to all application program.

These options are :

Option number        Option name in MXD.OPT and action.

-1             LARGLINE   line size in 80/132 for NO/YES (or

0/1).

0             SHORTLST   short listing if YES ( or 1 ).

as this :

OPTION(LARGLINE) = YES;  OPTION(SHORTLST) = YES;

enables the 132 columns mode for the listing.

The default is YES for the option SHORTLST for the MXDLSQ  program  and

the  LARGLINE  default  is  NO, but, then SHORTLST is negated (NO value) the

option LARGLINE is always reset to YES for the present version of MXDLSQ.

CHAPTER V

PHYSICAL PRIMITIVES OF MXD.

V.1  THE UNIT CELL DEFINITION STATEMENT.

There are two Primitives to form a cell definition statement:

CELL <c#a> [,<c#b> ,<c#c> ,<c#alpha> ,<c#beta> ,<c#gamma> ] ;

RCELL <c#a> [,<c#b> ,<c#c> ,<c#alpha> ,<c#beta> ,<c#gamma> ] ;

The parameters of CELL or RCELL primitives are the direct or reciprocal

unit  cell  parameters.   The  alpha,  beta and gamma values can be given in

decimal degrees or cosine value.  For the primitive  RCELL,  the  unit  cell

must  be  a Rhombohedral unit cell.  These statements are used to define the

unit cell and also, for magnetic and/or  modulated  structures  the  working

space axis X, Y, Z.

If we denote U o V as the vectorial product of U by V.

For the CELL statement form :

X // a,

Y perpendicular to X in the (a,b) plane,

and Z = X o Y.

For RCELL statement form :

X + Y + Z // (111),

X o Y, Y o Z, and Z o X are three vectors inside the plane [111].

These two definitions of the Direct Unitary  and  Orthogonal  reference

all symmetry relation are easy to write, specialy for Rhombohedral system if

you use the RCELL primitive form.

In a MXD data file you must have only one CELL or RCELL primitive.

PHYSICAL PRIMITIVES OF MXD.                                             Page V-2

The unit cell definition statement performs :

-Computing of reciprocal or direct unit cell parameters.

-Build the TMD and TMR matrixs defined below :

(X,Y,Z) = ((TMD)) * (x,y,z).

(rh,rk,rl) = ((TMR)) * (H,K,L).

Where the vector (x,y,z) is in fraction of unit cell parameters,

the vector (H,K,L) is a reciprocal lattice vector,

the vector (X,Y,Z) is in angstroem(s),

and the vector (rh,rk,rl) is in 1/angstroem(s).

The next numeric symbol are also defined:

$A,          $B,       $C,

$ALPHA,   $BETA,   $GAMMA       are the direct unit cell parameters,

$#A,      $#B,     $#C,

$#ALPHA, $#BETA, $#GAMMA       are the reciprocal one's,

$TMD11, $TMD12,$TMD13,.. and so one ... until $TMD33

are the matrix elements of TMD,

$TMR11, $TMR12, ... until $TMR33

are the matrix elements of TMR.

All these parameters are automatically declared by  MXD  on  the  Unit  Cell

Definition Statement execution.  This virtual declaration is equivalent to:

PARAM <ident> = <c#exp> declaration.

V.2  DECLARATIONS AND ASSIGNEMENT STATEMENTS.

The PARAM  and  VARIABLE  primitives  are  used  to  define  one  (or  many)

identifier(s)  as  (a) parameter or variable identifier(s).  A Variable is a

_______ _____            _________ _____

numeric identifier with an initial value but Not an evaluable value  because

the  MXD  Compiler  cannot  know  the actual value of a MXD variable.  A MXD

Parameter can be:

A constant string expression <s#anything> to use of Compiler.

A constant numeric  expression  <c#anything>  to  use  by  Compiler  or  the

application program.

A constant array definition to use has form factor table.

A variable expression <#anything> for use by the application program.

PHYSICAL PRIMITIVES OF MXD.                                             Page V-3

V.2.1  Declaration And Assignement Of Variable.

A variable is a numeric object used as variable  by  the  least-squares

programs,  the  VARIABLE declaration creates (a) variable identifiers .  The

variable assignement can be used to change the initial  value  and/or  sigma

(given in the related declaration).  The variable declaration as this syntax

:

VARIABLE <ident1> [ = <c#v1> [: <c#s1> ] ]

[, <ident2> [ = <c#v2> [ : <c#s2> ] ],  ... ] ;

Where <c#vn> is the initial value of the n'th variables, and <c#sn>  is  the

related one initial sigma.

Define ident1, ident2, etc ...  as VARIABLE name.

The initial value and sigma are zero (by default).

Each Variable must be declared by a VARIABLE  STATEMENT  declaration  before

any reference to it.

One VARIABLE statement can be defined one or many Variable identifier.

The variable assignement changes the  initial  value  of  a  variable.   Its

syntax is :

<vident> = <c#init_value_expr> [ : <c#init_sigma_expr ] ;

V.2.2  Declaration And Assignation Of Parameters.

The parameter declaration is as below :

PARAM <proposition1> [, <proposition2>, <proposition3>  ... ] ;

And an assignement of constant parameter syntax is :

<proposition> ;

A <proposition> can be :

<ident> = <s#expr>  { for string declaration/assignation }

<ident> = <c#expr>  { for numeric constant declaration/assignation }

<ident>( <c#org>, <c#step> ) =

(< list of <c#exp> >); { for numeric array }

<ident> = < var-expr>  { for variable parameter declaration only }

PHYSICAL PRIMITIVES OF MXD.                                             Page V-4

In string constant case.

<ident> = <s#exp>  to define a string constant parameter.

Example :

PARAM DATE = ' 05-OCT-84 '; { define DATE as the string ' 05-OCT-84

';

TITLE DATE!!' MY DATA FILE '; { use the date identifier

to set the title as :}

{   TITLE ' 05-OCT-84  MY DATA FILE '; }

In numeric constant case.

<ident> = <c#exp>  to define a numeric constant parameter.

Example :

PARAM BU = 0.85; { set BU as the fermi length of U atom }

In numeric variable case.

<ident> = <#vexp>  to define a variable parameter.

Example :

PARAM MFE3 = - ( MFE1 + 2 * MFE2 ); { if FE1 is in 4A site,

FE2 is in 8J site,

FE3 is in 4B site,

fixe  the resultant moment

to zero }

This is the solution to allow any constraint relation.  The  assignation

statement  is  illegal  for  variable parameter because we must not change a

variable parameter definition.

In numeric array constant case.

<ident>( <c#org>, <c#step> ) = ( <c#exp1> [, <c#exp2>,

<c#exp3>,etc... <c#expn> ] );

To define an array parameter.

In this last case, org is the subscript origine value, step is the  step  of

the  subscript,  and  exp1,exp2...expn  are the succesive value began at the

subscript org.

Example :

PHYSICAL PRIMITIVES OF MXD.                                             Page V-5

{ X-RAY FORM FACTOR OF THE IRON , SOURCE : INTERNATIONAL TABLE }

PARAM FEFRX(0,0.05) = (

26.00, 25.30, 23.68, 21.85, 20.09, 18.40, 16.77 );

{ THIS TABLE IS CONTINUED WITH A STEP OF 0.1 (1/Angstroem) }

{ THIS IS REALIZED WITH AN ASSIGNATION STATEMENT }

FEFRX(0.4,0.1) = (13.84, 11.47, 9.71, 8.47, 7.60, 6.99,

6.51,  6.12, 5.74, 5.39, 5.03, 4.69);

In this Example we can see the use of a Form-factor definition and  The  use

of  an  assignation to append some new value at an existing array parameter.

The org value in assignation can be overlap the  existing  definition,  then

the  old  end  of  list  is  discarbe  to have a conform definition, with an

increased subscript.

A form factor reference has two forms :

A variable form as <ident> and a subscripted form as <ident>(<c#subscript>),

The variable form is used to define a scattering factor,  the  interpolation

is  performed  by  the  DATA  REDUCTION  statement.   This  reference as the

attributs of a variable expression.

The subscripted form is used to give the interpolated  value  as  a  numeric

constant  value,  in  this  case  the  subscript  must be a numeric constant

expression.

V.3  VARIABLE MANAGEMENT STATEMENTS.

V.3.1  FIXED And UNFIXED Statements.

We can lock one or any number of variable with the statement :

FIXED <ident1> [, <ident2> [, <ident3> ... ] ] ;

and unlocked some already locked variable with the statement :

UNFIXED <ident1> [, <ident2> [, <ident3> ... ] ] ;

In the two cases <ident1>, <ident2> ...  are variable names.  By  default  a

variable is always not locked.

V.3.2  Variable Range Specification.

It can be usefull to specify an allowed range to  a  variable.   The  LIMITS

statement  enable  the  user  to  specify  a particular range at one or many

previously defined variables.

PHYSICAL PRIMITIVES OF MXD.                                             Page V-6

The possible syntax are :

LIMITS <ident>, <c#v1>, <c#v2> ; { for one variable }

LIMITS <c#v1>, <c#v2>, <ident1> [, <ident2> ... ] ;

Here v1 and v2 constant expressions can be :

v1 = lower limit, v2 = upper limit

v1 = modulo value, v2 = lower limit.

Example :

LIMITS V, 3, 12 { V must be in the range [3,12] };

{ A1, A2 and A3 are angle in the range [-180,+180[ }

LIMITS 360,-180, A1, A2, A3;

V.3.3  Diagonal Block For Least-square Program.

For Many purposes, it can be usefull to suppress the correlation between two

(or more) groups of variables.  This is realized by the next below statement

:

LSQBLOCK(<s#blkname>) = <c#dampening_factor_of_this_block>

[, <#fdyn> ] ;

The blkname must be  unique  in  all  blkname  in  the  data.   The  general

expression <#fdyn> can be used to give a dynamic dampening correction factor

that can be used to  define  a  convergence  acceleration  algorithme.   The

effective dampening factor is the product of two once if <#fdyn> is given.

PHYSICAL PRIMITIVES OF MXD.                                             Page V-7

It is also  the  method  to  set  a  dampening  factor  for  a  full  matrix

refinement.  The use is demonstrates below :

... variables definitions of the first lsq-block ...

LSQBLOCK('BLOCK_1') = 0.5; (* dampening factor is 0.5 *)

... variable definitions of the second lsq-block ...

LSQBLOCK('BLOCK_2') = 0.8; (* dampenig factor is 0.8 *)

... next variable definition ...

The last  variables  are  in  any  defined  lsq-block,  in  this  case,  the

least-square program will create an lsq-block as below :

LSQBLOCK('BLK.MAIN') = 1.0;

V.4  THE ATOM DECLARATION STATEMENT.

ATOM

or   (<s#name>) = [ <#rdif>, <#idif>, <#pop>, <#x>, <#y>, <#z>,

CATOM

and the other successive parameters :

<#dbiso> ] ;

or in anisotropic form :

<#u11>, <#u22>, <#u33>, <#u12>, <#u13>, <#u23> ] ;

With the ATOM primitive,  the  coordinates  x,y  and  z  are  in  unit  cell

parameter fraction, and with the CATOM primitive in angstroem(s) in the work

reference as defined by the CELL definition statement.  The dbiso  parameter

must  be  the  isotropic U (default mode) or if the statement "DEBYEMD ;" is

used, the DEBYE FACTOR B.  The anisotropic Uij parameters as in angstroem**2

unit,  along  the  crystallographic reference with ATOM primitive, and along

the work reference with CATOM.

All the atome name <s#name> must be unique in the atom data list.

All the parameter fields can be omitted as this :

Example :

ATOM('FE3+1') = ,,,x,y,z; { define an fe atom for a pure

magnetic structure determination }

PHYSICAL PRIMITIVES OF MXD.                                             Page V-8

V.5  THE WAVE VECTOR DECLARATION STATEMENT.

A wave vector  is  used  to  describe  a  magnetic  or  structural  periodic

modulation.  A good example of this is the helimagnetism.

The wave vector declaration is realized with the WAVEVECT primitive as below

:

WAVEVECT(<s#name>) = <c#qx>, <c#qy>, <c#qz> [, <cl#r> ] ;

Where qx, qy, qz are the components of the wave  vector  in  the  reciprocal

lattice reference.  If 'R' is ommitted the wavevector

The vector name must be unique in all wave vector name.

V.6  THE POLARIZED DIRECTION DECLARATION STATEMENT.

A polarization magnetic field direction must  be  given  for  all  polarized

neutron computing :

This declaration is realized with the NPOLADIR primitive

as below :

NPOLADIR(<s#name>) = <#Hx>, <#Hy>, <#Hz>

[ <#Ef+>, <Ef->, <#Cpol> ] ;

Where Hx, Hy, Hz are the componants of the magnetic  field  in  the  working

space  reference  system.   The Ef+ and Ef- are the polarization efficiences

defined as this:

When polarization is +, The flux is really :

K * [ Ef+ neutrons_up + (1 -Ef+) neutrons_down ]

When polarization is +, The flux is really :

K * [ Ef- neutrons_down + (1 -Ef-) neutrons_up ]

The Cpol parameter is the extinction correction coefficient expression.  The

parameters  can  be  general  expression  to  can be adjust with the current

orientation of the crystal.

The direction name must be unique in all direction name.

V.7  THE MAGNETIC MOMENT DECLARATION STATEMENT.

The declaration of a magnetic moment has the below syntax :

PHYSICAL PRIMITIVES OF MXD.                                             Page V-9

MOMENT( <s#name>, <s#atome> [ , <s#wave> ] ) =

[ <#magndif>, <#rmx>, <#rmy>, <#rmz>,

<#imx>, <#imy>, <#imz> ] ;

Where magndif is the magnetic scattering, rmx,rmy and rmz are the real  part

of  the  magnetic moment, and imx,imy,imz the imaginary part of the magnetic

moment only usable for the modulated magnetic structure.

The magnetic moment components are in Bhor Magneton unit.

The moment name must be unique in all the moment list, <s#atome> is the name

of  the  atom  where  the  magnetic  moment  is located, and <s#wave> is the

related wave vector name, if the magnetic moment is modulated.

It is possible to define any number of moment on the same atom  to  describe

any multi wave-vector structure.

The magnetic moments are always given in the work reference.

V.8  THE MODULATED DISPLACEMENT DECLARATION STATEMENT.

The declaration of a structural modulation has the below syntax :

MDSDSP( <s#name>, <s#atome>, <s#wave> ) =

[ <#mpop>, <#phpop>, <#rux>, <#ruy>, <#ruz>,

<#iux>, <#iuy>, <#iuz> ] ;

Where mpop and phpop are the magitude and phasys of a modulated  population,

and rux, ruy and ruz are the real part, and iux, iuy, iuz the imaginary part

of the modulated displacement.

The displacements is always given in the work  reference  and  in  angstroem

unit.

The moment name must be unique in all the moment list, <s#atome> is the name

of  the  atom  with  the  this  modulation, and <s#wave> is the related wave

vector name.

It is possible to define any number of modulation  with  the  same  atom  to

describe any multi wave-vector structure.

PHYSICAL PRIMITIVES OF MXD.                                            Page V-10

But in this last case the actual implementation of the MXDLSQ program cannot

work in multi-wave-vector case if the modulation play on the atom position.

CHAPTER VI

SYMMETRY MANAGEMENT STATEMENTS.

In MXD the symmetry information is optional.  You can always work in the  P1

or  P-1  mode (if you have an inversion center at the origine) by the use of

literal constraints in the ATOM/CATOM statements, you have any  benefice  to

use  the  symtry  operation  except  if the number of described atom is very

reduced in your data.  When you use the  SYMTRY  or  CENTER  statements  the

occupancy probability (population) field in the ATOM/CATOM statement must be

the product of the true occupancy probability by the  site  multiplicity  as

with  a  standard  BUSING  -  LEWY program.  If all atoms are in the general

position, then the time consuming is lightly less than when you describe all

equivalents  atoms  directly.   The centered mode is more efficient because,

the computing time is approximatively divide by two (2) when it is used.

You have an other limitation of the symmetry statements.  If an atom  has  a

magnetic  moment or has a modulated population or position, then you can not

use SYMTRY and  CENTER  information  for  it,  beacuse  you  must  define  a

particular  name  for  each  equivalent  atome.   In  this case the magnetic

moments or the modulations can have any relations.  The examples at the  end

of this chapter show why do a good usage of symtry statements.

VI.1  THE LATTICE STATEMENT.

The syntax is :

LATTICE <l#lattice> ;

The supported lattice name are P, A, B, C, I, H, F where H is the letter  to

indicate  the  use  of  an hexagonal lattice with the rhomboedric extinction

-H+K+L=3n .  This statement is used two suppress the normal computing of the

atomic  structure  factor  for  not modulated atoms if the reflection is not

allowed in the given lattice.

SYMMETRY MANAGEMENT STATEMENTS.                                        Page VI-2

VI.2  THE CENTER STATEMENT.

The syntax is :

CENTER [<c#exp>];

{ CENTER ; is equivalent to CENTER YES; or CENTER 1; }

The CENTER option set the mode centered if the constant parameter  value  is

>=  0.5  (YES  logical  value) and release it if the parameter is < 0.5 ( NO

logical value ).  If many CENTER statement are given the last it define  the

centered mode (YES or NO).

The use of  the  center  statement  is  recommanded  because  it  is  always

advantageous in cpu time conssuming.

VI.3  THE SYMTRY STATEMENT.

The syntax is :

SYMTRY( <s#name> ) = <c#m11>, <c#m12>, <c#m13>, <c#tx>,

<c#m21>, <c#m22>, <c#m23>, <c#ty>,

<c#m31>, <c#m32>, <c#m33>, <c#tz>;

The name of each symtry operator must be unique in the list  of  all  symtry

operators.   m11...m33  are  the  matrix  elements  and  tx,  ty, tz are the

translation.

Example :

SYMTRY('4.1') =  0, -1,  0,    0,  { helicoidal axis (4 | 0 0 1/4) }

1,  0,  0,    0,

0,  0,  1,  1/4;

VI.4  THE GENSPACE STATEMENT.

The syntax is :

GENSPACE ;

SYMMETRY MANAGEMENT STATEMENTS.                                        Page VI-3

You have two possibilities:  you enter all symmetry operators (but  not  the

operators  that are equivalent by a lattice translation) or to give a set of

generator operators.  This last case use the statement GENSPACE.

Example :

{ space group is Fm3m }

LATTICE 'F'; { lattice is F: h+k=2n, k+l=2n, l+h=2n }

CENTER;      { centered space group }

SYMTRY('3[111]') = 0, 1, 0,  0,

0, 0, 1,  0,

1, 0, 0,  0; { axis 3 along 111 }

SYMTRY('4[001]') = 0,-1, 0,  0,

1, 0, 0,  0,

0, 0, 1,  0; { axis 4 along 001 }

GENSPACE;

VI.5  SOME CONSIDERATIONS ON THE USE OF SYMMETRY INFORMATIONS.

The symmetry management statements of MXD enable the user to work in a  mode

similary  to the ORFLS.  When these statements are used the non magnetic and

non  modulated  atoms  (neither  referenced  in  MOMENT  statement,  neither

referenced  in  MDSDSP  statement), the statistic occupancy must include the

local symmetry order (or site multiplicity).   The  mangnetic  or  modulated

atoms  are not sensitive to the SYMTRY and CENTER statements, then, they can

be described completly always.

In other side, it  has  not  evidence  that  the  symmetry  statements  save

computer  time  when  you have a little number of atom in a very symmetrical

space group.  So it is a good pratique to use the SYMTRY statement only when

the   description   of  the  structure  (in  term  of  number  of  ATOM)  is

significantly shortlest.

A special consideration must be explain for the CENTER statement because  it

saves  always  some cpu time if you have some not modulated and not magnetic

atoms (except in the case where the unique atom is on the origine (where  is

the center).

The CENTER statement modifies the computing of the structure factor but  the

SYMTRY  operator  are  applied at each not magnetic or modulated atom to get

each symmetrycal equivalent one.   When  GENSPACE  is  used  and  CENTER  is

specified  it  change the space groupe to an acentric equivalent group where

all the symmetry operators are different from the center operator.

SYMMETRY MANAGEMENT STATEMENTS.                                        Page VI-4

Example :

{ we want describe the group Pm3 }

CENTER;

SYMTRY('3111')  =  0, 1, 0,   0, { give the 111 threefold axis }

0, 0, 1,   0,

1, 0, 0,   0;

SYMTRY('mx')    = -1, 0, 0,   0, { give mx miror }

0, 1, 0,   0,

0, 0, 1,   0;

GENSPACE;                         { generate the group }

In this example the space group Pm3  will  be  generated  but  in  fact  the

operator  mx,  my,  mz  and  the four threefold axis will be generated.  The

binary axis 2x, 2y and 2z will not be generated because they are  equivalent

to the mx, my and mz miror if the inversion center is handle as the identity

operator.

CHAPTER VII

THE EXPERIMENTAL DATA MANAGEMENT STATEMENTS.

The DATA management statements are used to define the experimental  data  in

term of DATA COLLECTION.

A DATA COLLECTION is a set of experimental data with a  identification  name

and an unique scale factor.

The complet DATA statement enables the user to define an extinction rule for

each  reflections,  and  describes  the input data file to accomodate with a

large range of experiment output.  So the data  collection  statement  is  a

versatil  data  reduction process.  The main restriction is to have only one

input line for one h k l reflection.

we have also a simplified  form  of  the  data  statement  to  declare  your

intention to use a previously defined data collection.

VII.1  DATA COLLECTION DEFINITION STATEMENT.

The syntaxe of the DATA statement is :

DATA(<s#name>,<#scale> [,<formula> [,<formula> ...]])

<id1>,<id2>,<id3>,...<idn>;

data line or data directive

...........................

...........................

END;

An alternate form of the DATA statement to used the extinction correction is

:

DATA(<s#name>,(<#scale>,<#cfn2>,<#cfm2>,<#dynw>)

[,<formula> ...]) <id1>,<id2>,

THE EXPERIMENTAL DATA MANAGEMENT STATEMENTS.                          Page VII-2

..<idn>;

...........................

...........................

END;

<#scale> parameter is the sclae factor expression.

<#cfn2> is the nuclear coefficient correction.

<#cfm2> is the magnetic coefficient correction.

<#dynw> is the dynamic weight factor.

The data line are single line with some number with space(s), tabulation  or

comma separation.  Two successive comma enable user to skip a data field and

a semicolon end the data line; in this case the data parsing of this line is

terminated, the next reflection in take from the next line.

All comment are allowed.

{ h  k  l   f2   sg}

1 -2  3  12.5  .8 { is a data line, the format is free }

1,,,8; { is an other data line with

h = 1, k and l are defaulted( 0 is normal default) f2=8,

sg defaulted.

if no given default this is equivalent to : }

1  0  0   8.0  0.0

The <id1>,<id2> ...  <idn> fields are used to define the data line fields.

<idp> define the th'p field in a data line.

Each id has this syntax :

<ident> [= <c#defaultvalue>]

THE EXPERIMENTAL DATA MANAGEMENT STATEMENTS.                          Page VII-3

You have two type of field identifiers :

The Predefined identifiers used two define all MXD hkl data Characteristics.

The user previously defined field ( with the FFASSIGN statement) are handled

as additionnal predefined identifiers.

The temporary identifiers used as unused data field or temporary field.

All  predefined  identifiers  must  be  receive  a  value  during  the  data

evaluation.

This evaluation can be done automaticaly by a super default value,

By the explicite specification in an <idp> item,

or by a given formula.

The predefined field identifiers are :

super

identifier  default        nature

value

IH         0          H integer index of the reflection,

IK         0          K integer index of the reflection,

IL         0          L integer index of the reflection,

MUL        1          Multiplicity of the reflection (used with

RAY),

NQ         0          for satellite as: h = H + NQ*q,

NPOLA      0          neutron polarization specifier

possible value are :

0           none polarized,

1 or  +     for I+ = (I++) + (I+-)

2 or  -     for I- = (I--) + (I-+)

3           for I+/I- (flipping ratio)

5 or ++     for I++

6 or --     for I--

7 or +-     for I+-

8 or -+     for I-+.

SELNB      0           Select number to use with SELECT standard

function value in the range [0..31],

IS         1           Reflection flag in the range [-63..63]

abs(is) is the reflection category number

THE EXPERIMENTAL DATA MANAGEMENT STATEMENTS.                          Page VII-4

This category can be used for select

some class of reflection,

is = 0 => reflection is ignored,

is < 0 => this reflection is a part of a

reflection packet,

IS > 0 => this reflection is the final

reflection in a packet

with given observation value

or a single crystal reflection,

SF         0.0         Observed Structure factor for structure

factor data,

F2         0.0         Observed Squared structure factor (for

F**2 data ),

RAY        0.0         Observed Lorentz corrected Intensity

( for packet powder data),

SF, F2, and RAY are mutually exclusive,

SG         0.0         Sigma of the observation,

WE         1.0         Least square Weight of the observation.

<user field ident>   0.0         Any usage they are stored in a form factor

table of the reflection binary data

record.

The temporary field are always the 0 default value if not specified.

And additional symbol is defined in the DATA reduction statement.   This  is

the  symbol  $NPT that denotes the current reflection number.  It is started

from 1 for the first reflection and can be used in the <formula>e.

The <formula> are used to give a value to a predefined or user  defined  (by

FFASSIGN  statement)  field, they can use the any <c#expr> with reference of

OTHER predefined, user defined or temporary fields.

The syntax is :

<predefinedident> = <c#expr>

The DATA Directive can be give other information to the DATA  reduction

processing as :

Include an other file in the current data stream with the same

syntax as the INCLUDE statement :

INCLUDE <s#filename> [,<opt1> [, <opt2>, ... ] ];

THE EXPERIMENTAL DATA MANAGEMENT STATEMENTS.                          Page VII-5

Simulate a end of file before the data end of file :

ENDFILE;

Specify the current wave vector with :

WAVEVECT(<s#existing_wave_vector_name>);

Specify the current polarization neutron magnetic field direction

with :

NPOLADIR(<s#existing_polarized_direction_name>);

Example 1 :

{ single crystal F data collection reduction }

{ the data in the file have the form: h k l f sigma }

DATA('Ourdata',Scale,WE=1/SG) IH, IK, IL, SF, SG;

{ Get the data in two successive other files }

INCLUDE 'DATAFILE1.DAT';

INCLUDE 'DATAFILE2.DAT';

END;

Example 2 :

{ single crystal data in F2 and collection wanted in F }

DATA('F2 coll.',Scale,WE=1/SG, SF=SQRT(F2), SG = SGLU/ABS(2*F2LU) )

IH, IK, IL, F2LU, SGLU;

...

END;

Example 3 :

{ Powder packet data reduction with $ABS absorption correction }

DATA('POWDER1',SCALE*$ABS, WE=1/SG) IH,IK,IL,MUL=2,IS=-1,RAY,SG;

1 2 3 8

2 3 0 4 1{IS} 125{F2}, 2.5{SG}

...

END;

THE EXPERIMENTAL DATA MANAGEMENT STATEMENTS.                          Page VII-6

EXAMPLE 4 :

{ Modulated structure with two wave vectors q1 and q2 in powder }

DATA('data20k',SCALE,WE=1/SG,MUL=2) IH,IK,IL,NQ,IS=-1,RAY,SG;

{ Nuclear reflection }

1 2 0  0  1  123 34

{PACKET OF TWO magnetic SATELLITES with the two wave vector }

WAVEVECT('Q1');

2 0 0  1

WAVEVECT('Q2');

3 0 0 -1  1   12.5 1 { packet with the two satellites }

1 1 1 -1  1    23 2 { The current wave vector is Q2 }

...

END;

EXAMPLE 5 :

{ Get data from 3 point mesure, the scan time is equal to total

background count time in this example }

{ Use An Extinction correction }

(* nleft and nright are the counts of

background and nscan the scan count *)

DATA('PHILIPS',(SCALE,E$Y),

WE = 1/SG { standard weight },

SG=(NSCAN+NLEFT+NRIGHT).5 { sigma },

F2=NSCAN-(NLEFT  +NRIGHT)) { intensity }

) IH, IK, IL, IS=1, NLEFT, NSCAN, NRIGHT;

INCLUDE 'MYFILE.DAT';

END;

VII.2  DATA COLLECTION REFERENCE STATEMENT.

If the number of reflection is large, it can be advantageous to  reduce

the data collection in bynary form only the first time.

In this case we can use the short DATA statement as below :

DATA(<s#name>) = <#scale> [,<#cfn2>,<#cfm2>,<#dynw>];

VII.3  CLRDATA STATEMENT.

The syntax is :

THE EXPERIMENTAL DATA MANAGEMENT STATEMENTS.                          Page VII-7

CLRDATA ;

This statement clear all previously  existing  DATA  collection.   This

statement,  if  used,  must  be  given before all DATA statement.  It can be

simulated  by  a  delete  of  the  file  "MXDBDD.DDI"  (  on   the   VAX/VMS

implementation ).

This statement do not delete any file but allow MXDCMP to supersed  the

old DATA files ("MXDBDD.DDI" and "MXDnnn.BDA" in VAX/VMS system).

VII.4  MANAGEMENT OF FORM FACTOR TABLE IN BINARY DATA FILE.

The statement FFASSIGN can be used  to  allocate  a  particular  column

number  at a predefined form factor or to define a new predefined data field

in a this allocated column.

The syntax is :

FFASSIGN <ident>[:<c#clnm>] [,<ident>[:<c#clnm>] ...] ;

If the <ident> is already defined table parameter (existing form factor

by  example)  a  column number is allocated only and the data statement will

use it to store the sin(theta)/lambda interpolation of this table.

If clnm is specified and positive, it is the column  number  (in  range

[1..12]),  else if it is specified with negative value, the column number is

allocated from the end of table (column number = 13-clnm).  It is convenient

if the maximum column number is unknown (presently 12).

If clnm is not specified then the column given is the first free column

in the increasing order.

VII.5  MAGNETIC SPECIFICATION STATEMENT.

If you have different data collection for different experiment  on  the

same  compound.   By  example  x-ray an neutron.  You hope that MXD comput a

magnetic structure factor only for the neutron data.  But MXD  do  not  know

that  is  the x ray data collection and that is neutron data collection, and

probably  compute  the  magnetic  structure  contribution  for   both   data

collections.

To avoid this, you must use the MAGNETIC statement :

MAGNETIC <ch0> [,<ch1> ....  <ch31>];

where <chn> is a choice as {YES | NO } or { 0 | 1 }.

The different specification ch0 to ch31 are given for each used value of the select  number SELNB (defined in the DATA statement). If a data collection has a choice NO, then the magnetic contribution is not computed, and alternatly the choice YES or 1 enable MXDLSQ to compute the magnetic contribution.

Example :

{ set rdf to b for neutron and f + delta f' for x-ray }

PARAM FERDF = SELECT(bfe,fefrx+fedfp),

{ set idf to nothing (0) for neutron and delta f" for x-ray }

FEIDF = SELECT(,fedfs);

...

ATOM('Fe1') = FERDF, FEIDF, ... ;           { to define an atome }

...

MOMENT('Mfe','Fe1') = ...         ;           { to define a magnetic moment

}

...

MAGNETIC YES, NO;     { to set no magnetic computing with x-ray }

...

DATA( ... SELNB = 1 {x-ray} ...) ...;

...

END;

DATA( ... SELNB = 0 {neutron} ...) ...;

...

END;

VII.6  WARNING FOR PREVIOUSLY DEFINED DATA COLLECTIONS

When you use some previously defined DATA collection you can have some important unwanted side effect. The binary data records have twelve (12) fields to hold twelve (12) user defined field values or interpolated form factors. The form factor column  (in range 1..12) can be allocated automatically or by the FFASSIGN statement. The data reduction process does the interpolation of all allocated form factor and the automatic allocation is involved by the reference of an unallocated form factor table (in general by the atom statements that use the form factor tables). If you change some thing in the atom (or magnetic moments) order without do a new complete data reduction (by the use of a data reduction reference statement), then you can have some data binary file(s) ("MXDnnn.BDA" in VAX/VMS implementation) with the column value not in agreement with the new form factor allocation. In this case the computing realized by MXDLSQ is bad because  the  form  factor can be permuted or not defined. The recommended procedure is to use the  FFASSIGN statement to define all used column is the binary data file(s).

CHAPTER VIII

EXPRESSIONS AND SYMBOLS IN MXD.

At the initialize time the  data  compiler  MXDCMP  define  a  list  of

standard  function  and  parameters to enable the user to write any relation

dependence with the current computing environment.

Example :

{ Extinction as defined in SHELL-X crystallographic system }

PARAM $LAMBDA = 0.707 {X - RAY Mo };

VARIABLE E$X = 0; { define an extinction variable }

LIMITS 0, 1, E$X; { limits the extinction variable excurtion range to 0..1}

PARAM { give the formula of the extinction coefficient for each refl. }

E$Y = (1 - 0.0001 * $FN2 * E$X / $SITHSL / $LAMBDA)**2;

{ here $FN2 is the squared atomic structure factor of the current

reflection, and $SITHSL is the Sin(Theta)/Lambda of the current

reflection }

In the previous example we use the predefined  symbol  $SITHSL  as  the

value  of  the  1/2d  (=  sin(Theta)  /  Lambda  ) of the currently computed

reflection.  We use also the symbol $FN2 as  the  Squared  atomic  structure

factor  for  the  current  reflection.   This last symbol is generally atomic

parameter dependant, and MXDLSQ will know compute all  partial  derivatives  of

the newly defined variable parameter automatically.

The general convention for the MXD reserved name is to have the special

character $ in the identifier name.

An other category of predefined symbol is the  MXD  standard  functions

set.

EXPRESSIONS AND SYMBOLS IN MXD.                                      Page VIII-2

The user must be remark that all these symbols are not reserved  (as  a

keyword)  but  there  use for other purpose can be generate some compatibly

problem with the use of the standard function.

The MXD expression management is a set of rules to drive  all  formulae

interpretation.

The first rule is :

MXDCMP try to realize all written operation at the compile time.  If an

operator  or  a function is not compatible with these actual parameter kinds

MXDCMP try to convert these  expression  in  the  desired  type.   An  error

message is edited if this conversion is not possible.

The second rule is :

The operator or function are the drivers to applied a type  conversion.

By example + is a arithmetic operator for addition, so if one is (or the two

operand are) not numeric, then the appropriate conversion are generated.

The distinguished type are :

1.

The String constant type that can be used at the compile time only.

2.

The Numeric constant type, usable by MXDCMP and MXDLSQ.

3.

The Variable parameter type, usable by MXDLSQ only

EXPRESSIONS AND SYMBOLS IN MXD.                                      Page VIII-3

By MXDLSQ we must understand any application program.

VIII.1  THE MXD FUNCTIONS.

VIII.1.1  Standard Function Of MXD.

The defined transcendent functions are :

SIN(<#e>)           sine( angle in degrees).

COS(<#e>)           cosine( angle in degrees).

TAN(<#e>)           tangent( angle in degrees).

ASIN(<#e>)           arcsine( result in degrees).

ACOS(<#e>)           arc cosine( result in degrees).

ATAN(<#e>)           arctangent( result in degrees).

ATAN(<#eb>,<#ea>)   phases of the complex number ea + i* eb

( result in degrees).

EXP(<#e>)           exponential.

LOG(<#e>)           natural logarithm.

SQRT(<#e>)           square-root.

TANH(<#e>)           hyperbolic tangent.

ABS(<#e>)           magnitude of <c#e>.

MODULO(<#e1>,< e2>)           return the modulo of <#e1> by <#e2>

ROUND(<#e>)           the nearest integer value of <c#e>.

The next standard function cannot be  used  in  a  constant  expression

because  it  can be evaluate only by the executor process in the application

program (MXDLSQ).

SUMM(<idx_ident>,<#eb>,<#ee>,<#es>,<#exp>) to comput the summation

of the expression exp when the scalar idx_ident run from

eb to ee by step of es.

EXPRESSIONS AND SYMBOLS IN MXD.                                      Page VIII-4

This function can be used for integration purpose.

INTSEL(<#esel>,<#e0> [,<#e1> ...  <#e31>]) to comput the nearest

integer n of the esel expression and return the en'th expression.

SELECT(<#e0> [,<#e1> ...  <#e31>]) to return the eSELNB'th expression.

SUMHKL(<#exp>) return the summ on all selected reflection of the

parameter expression.

The next standard function can be used only  with  constant  expression

parameters because its are computed directly by the data compiler.

The main usage of its, is in the MACRO directive context.

SUBSTR( <s#string>, <c#istart>, <c#len>)

Return the sub-string of string begining at the istart character

and with len characters of size.

STRING( <c#value> [, <c#format> ] )

Take the nearest integer of value, convert and return the

equivalent string. If format is given, it is the number of digit(s)

the unsignificant (at left) digits are the zero (0) digits.

LENGTH( <s#string> )

Return the length (in character) of the given string.

INDEX( <s#string1>, <s#string2> )

Return the position (in character) of the sub-string string2

in the string string1 (first occurance only). If not found the

result is 0.

NUMBER( <s#string> )

Return the integer value of the string.

EXPRESSIONS AND SYMBOLS IN MXD.                                      Page VIII-5

DEFINED( <identifier> )

Return an integer value to flag the defined or not defined

identifier.

The returned value are :

-1                 Undefined Macro parameter (with no actual).

0                 Undefined identifier (not declared).

1                 Defined identifier.

2                 Numeric constant.

3                 String constant.

4                 Legal operator.

PARAMREF( <identifier> )

Return the sequence number of the Variable Parameter identifier

or 0 if the identifier is not defined (or declared) or this is

not a Variable Parameter. This function must be used with the

DSPLPAR option of MXDLSQ and must be used to give a Variable

Parameter sequence number. That is the unique method to transfer

a parameter reference across the OPTION statement.

VIII.1.2  User Defined MXD Function.

MXD enable user to define some no standard function by the use  of  the

next statement ( a function_declaration ) :

FUNCTION <name_id> (<P1> [,<P2>...] ) = <#expr>;

Where <Pn> is :

{ <scalar_identifier> | <function_declaration> }

The function declaration  is  used  to  define  a  function  as  formal

parameter.  The user can not give a standard function as actual parameter of

a user defined function :

EXPRESSIONS AND SYMBOLS IN MXD.                                      Page VIII-6

Example :

FUNCTION INTEGR(A,B,FUNCTION F(X))  { to integrate f from a to b with

10 points}

=

SUMM(X { integration variable},

A {initial value},

B-(B-A)/20 {final value},

(B-A)/10 { step for summation },

(F(X) + F(X+(B-A)/10))*(B-A)/20 { the summation formula }

);

Warning :

The User defined function are convenient for the data edition  but  can

be  heavy  for  the  run  time.  Therefore it can be preferable to solve the

problems without to use the user function when it is not too complicated.

VIII.2  RESERVED AND PREDEFINED SYMBOLS.

MXD define a set of symbols to give access to  some  pertinent  dynamic

data during the least square fitting.

Mxd define also some other  symbols  to  keep  some  crystallographique

information.

VIII.2.1  Predefined Variable Parameters.

These symbol are not real Variable parameter and can not be used by the

PARAMREF function ( => DSPLPAR option of MXDLSQ).

The list of these parameter is given below :

EXPRESSIONS AND SYMBOLS IN MXD.                                      Page VIII-7

The values of the next parameters change for each reflection.

Name(s)           Usage

$H, $K, $L       Integer indexes of the current reflection.

They are the components of H in the reciprocal lattice.

$RH, $RK, $RL    Components of current H in the work space reference

system.

$HH, $KK, $LL    Components of current h in the work space reference

system.

$QX, $QY, $QZ    Components of the current wave vector in the work space

s.

$OBS             The current observed value SF, F2 or RAY.

$SIG             The current observation sigma.

$WEIGHT          The current static weight (as defined in the DATA stat.).

$SITHSL          The current 1/2d = sin(Theta)/ Lambda.

$HX, $HY, $HZ    The components of the polarization magnetic field in

w. syst.

$NPOLA           The current polarization state index as defined in the

DATA statement.

The values of the next parameters changes each symtry operator change.

They are usable to describe anisotropic form factor formulae.

$SH, $SK, $SL    The symmetry transformed of $RH, $RK, $RL.

The next parameters are derivats and can be used for extinction purpose.

$FN2             Squared Atomic (or nuclear) Structure factor Fn**2.

$FM2             Squared Projected Magnetic Structure factor FM**2.

$F2POLA          Partial polarized neutron intensity (I++,I--,I-+ or I+-).

$CALC            Computed value without scale factor (Sf, F2 or RAY).

The next parameters are NOT derivate and can be used only for informative

Variable Parameter computing (by example to use SUMHKL function).

$FNR, $FNI      Real and imaginary parts of the atomic structure factor.

$FMXR,$FMXI )

$FMYR,$FMYI  )  Components of the real and imaginary parts of the

$FMZR,$FMZI )   unprojected magnetic structure factor.

The nextparam parameters are defined to write some formulae of convergence

acceleration process.

$LCHI2, $CCHI2 The last and the current squared goodness of Fit.

$LMAXF, $CMAXF The last and the current likelihood function.

VIII.2.2  Predefined Constant Parameters.

The CELL ( or RCELL ) statement define a  set  of  constant  parameter.

EXPRESSIONS AND SYMBOLS IN MXD.                                      Page VIII-8

These  constant  can  be  used  for any usage, but the user would not change

there value by an assignation statement.

The angle are the Cosinus.

$A , $B , $C , $ALPHA , $BETA , $GAMMA   The direct unit cell.

$A , $B , $C , $ALPHA , $BETA , $GAMMA   The reciprocal unit cell.

$VOLUM and $VOLUM  The direct and reciprocal unit cell volume.

The matrix TMD and TMR are also defined with :

| x |     | X |     x, y, z  crystallographic coordinates.

|| TMD || * | y |  =  | Y | .

| z |     | Z |     X, Y, Z  work ref. syst. coordinates.

| h |     | Hr |     h, k, l coordinate in reciprocal

lattice.

|| TMR || * | k |  =  | Kr | .

| l |     | Lr |     Hr, Kr, Lr  work reference. syst coord.

The TMD and TMR matrix elements are defined as the constant parameters :

$TMDXX, $TMDXY, $TMDXZ, $TMDYX, $TMDYY, $TMDYZ, $TMDZX, $TMDZY, $TMDZZ

$TMRXX, $TMRXY, $TMRXZ, $TMRYX, $TMRYY, $TMRYZ, $TMRZX, $TMRZY, $TMRZZ

VIII.2.3  Reserved Parameter Names.

To be in agreement with the standard design of MXD some parameters  are

reserved  to  a  special  use.  You can use but the value would be really as

defined below :

$LAMBDA      Parameter               The current wavelength.

E$G          Variable or Parameter   Major extinction variable.

E$Y          Parameter               The current extinction coefficient.

CHAPTER IX

MACRO FACILITIES IN THE MXD LANGUAGE.

The MACRO  facilities  of  MXDCMP  enable  the  user  to  define  other

statement to extend the possibility, and/or take more easy the usage of some

complicated theory (by example :  Becker and Coppens Extinction correction).

IX.1  MACRO STATEMENTS.

The syntax is very easy to use, is it :

MACRO <macro_name> [ <P1>, <P2> ... <Pn> ];

< statement list of this macro >

ENDMACRO;

Where the Pi are the formal parameter names.

The MACRO can be used by the call :

<macro_name> [ <A1>, <A2> ...  <An> ];

Where the Ai are the actual parameter value.

A macro can be created an other macro if you use  the  MACRO  statement

inside :   This  is  a  nested  MACRO  definition.  A macro can call another

MACRO :  this is a nested MACRO call.  But a macro cannot call it-self.  The

recursive call of macro is not allowed.

MACRO FACILITIES IN THE MXD LANGUAGE.                                  Page IX-2

A macro can be deleted from the existing  (and  usable)  MACRO  if  the

macro is purged by the PURGE statement as this :

PURGE <macro_name1> [, <macro_name2> ...  ];

IX.2  LOCAL SYMBOL DEFINITION.

If some symbol have a limited use you can define  some  local  symbols.

The  local  symbol  are created in the current dynamic block and are deleted

when we go out of this block.   The  dynamic  block  begin  from  the  BEGIN

statement  and  are  finished by a END statement.  If a local symbol has the

same name that a more global symbol then the global symbol is not accessible

until the END statement where this local symbol is deleted.

The syntax is :

...  declaration of A, X symbol ...  { outer level block }

...

BEGIN

...  declaration of local symbol X,Y ...  { inner level block }

...  declaration of global symbol z ...

(* by example by :  VARIABLE 0 {The global lex level} Z=13.4; *)

...

{ each reference to X and Y is local }

MACRO FACILITIES IN THE MXD LANGUAGE.                                  Page IX-3

{ each reference to A IS more global }

...

END { the local symbol X and Y are deleted, but not Z };

...

{ the old X, A and the new symbol Z can be referenced }

...

Now we can describe the complet syntax of a declaration entry  for  the

PARAM, the VARIABLE, the FFASSIGN and the CONTRIBUTION statements.

[ <lex_level_index> ] <identifier> ...  { as decribe before }

The Lex_levelindex is an integer constant defined as this :

0 to create a global symbol.

n > 0 to create a symbol in the nested block of n'th outer  level  that

the  current  level.   If  we are in the block of deep of 4 and if n = 2 the

symbol should be created in the block 4 - 2 = 2.

Example : PARAM 0 $LAMBDA = 1.54 {Angstroem}; {$LAMBDA is global }

.F.S

This is usefull to write a MACRO (as the extinction macro) that creates

some global symbol (E$G, E$Y ...) but delete all these temporary symbols.

IX.3  CONDITIONAL COMPILATION OF A PART OF MXD SOURCE.

The conditionnal compilation statement (IF) is used to change  the  run

flow of the MXD data analyse without change the data file.

MACRO FACILITIES IN THE MXD LANGUAGE.                                  Page IX-4

The syntax is :

IF <c#expr> THEN <statement_list_if_true>

[ ELSE <statement_list_if_false> ]

ENDIF;

If  the   expression   expr   is   true   (or   >=   0.5),   then   the

statement_list_if_true  is compiled, else if the ELSE directive is used then

the statement_list_if_false is compiled.  The IF statement can be nested.

IX.4  THE REPEAT STATEMENT.

The syntax is :

REPEAT <statement_list_to_repeat> UNTIL <c#expr> ;

The statement_list_to_repeat is compiled in loop until  the  expression

expr  to  be  true  (  >=  0.5  ).  This is the unique macro loop defined in

MXDCMP.

IX.5  THE STOP ON DETECTED ERROR STATEMENT.

If a MACRO has detected a logical use error, it can be possible to stop

the process with a FATAL error flag to use the single statement :

ERROR ;

IX.6  THE MACRO LIBRARY MANAGEMENT OF MXDCMP.

You can create a file with a complet set of MACRO on the same  subject.

If this file is a list of MACRO definition without other MXD statement, then

this file is a MACRO LIBRARY.  MXDCMP is ready to  use  a  particular  MACRO

LIBRARY called the STANDARD MACRO LIBRARY.  The standard macro library is in

the VAX/VMS system the  file  "MXDLIB:MXDLIB.MXL".   The  user  can  load  a

MACRO FACILITIES IN THE MXD LANGUAGE.                                  Page IX-5

particular MACRO from the library to use the MACROCALL statement.

The MACROCALL statement as the following syntax :

MACROCALL <macro1> [, <macro2> ... <macro32> ] ;

You can load until 32 macro with one MACROCALL statement.

IF you want use other macro library you can specify :

MACROLIB <s#filename_of_macro_library> ;

Each MACROCALL statement search in the last specified macro library  in

first,  and  after  if some macro was not found, then search in the previous

macro library.  The last macro library is the standard macro library.

If we want suppress the search on the last macro library we can use the

single statement :

MACROLIB ;

This is used to reduce the  search  time  in  the  inappropriate  macro

library.

Example :

{ to use the EX$BECCOP macro for Becker-Coppens Extinction }

MACROLIB 'MXDLIB:EXTINC.MXL'; MACROCALL EX$BECCOP;

{ to use EX$BECCOP }

PARAM $LAMBDA = 0.89 { neutron wavelength }

EX$BECCOP 0 {neutron}, 0{isotrop}, 1{Type 1}, 1{Lorentz distribution};

PURGE EX$BECCOP; { purge now the macro to free memory requirement }

{ EX$BECCOP as define the E$G variable and

the VAriable Parameter E$Y that can be specified in the data statement

}

...

DATA('Our data', (SCALE,E$Y), ... ) ... ;

...

END;

MACRO FACILITIES IN THE MXD LANGUAGE.                                  Page IX-6

IX.7  THE CONTROLLED INPUT/OUTPUT OF MXDCMP.

MXDCMP can be work ask some information from then  controlled  terminal

(SYS$INPUT   in   VAX/VMS  system)  and  perform  a  limited  set  of  other

Input/Output operation.

IX.7.1  The Controlled Terminal Operations.

The terminal output statement is :

DISPLAY [ <P1>, <P2> ...  ] ;

The Pi field are :

<c#expr> [ :<c#n> [:<c#m> ]]

To output the numeric value in a standard pascal formats :

n <= 0, m <= 0     Default FLOTTING format.

n >  0, m =  0     Integer format In.

n <= 0, m >  0     Flotting format Em+7,m.

n >  0, m >  0     Fixed format Fn,m.

<s#expr> [ :<c#n> ]

To output a string expression, if n is specified

some space can be add to output exactly n characters.

At each use of the DISPLAY statement, one line  is  typed  out  on  the

terminal.

you can use DISPLAY; to output an empty line.

The REPLY statement is used  to  read  anything  from  the  controlling

terminal keyboard.  The syntax is :

REPLY <identifier> [ , ... ] ;

MACRO FACILITIES IN THE MXD LANGUAGE.                                  Page IX-7

Each identifier can be a number  or  a  string.   If  it  is  a  string

constant  identifier, all the line is stored in, else, it can be an constant

parameter, in this last case, one number  is  read  from  the  terminal  and

stored in the identifier.

The dialogue is obtaint by the combined statement :

DISPLAY <anything(s)> REPLY <a_thing>  [ , ... ] ;

In this case the input is doing  on  the  same  line  that  the  output

prompt.

IX.7.2  The Listing Outputs.

The output on the MXDCMP listing is obtaint by the  statement  WRITEMSG

that have the same syntax that the DISPLAY statement.

WRITEMSG <anything(s)> ;

IX.7.3  The User File Management.

MXD enable the user to read, write or extend until four (4) text files.

Each  file must be opened before any file operation.  The OPENFILE statement

attach a channel number ( in range 1..4 ) to the specified file that is used

by the READ, WRITE and CLOSEFILE statements.

To operate on an ASCII (text) file, the file must be previously opened.

This open operation is performed by the statement OPENFILE.

OPENFILE [ :<c#channel> ] <s#filename> [ , <l#mode> ];

Open the file on the I/O channel <cchannel> ( or on the I/O  channel  1

by  default).   The  <l#mode> is the operate mode for the file to open.  The

legal <l#mode> value are:

'Read'   To open an existing file for READ operations.

'Write'  If the specified file is not already existing,

the file is created before the open,

MACRO FACILITIES IN THE MXD LANGUAGE.                                  Page IX-8

then it is opened for WRITE operations.

'New'    To create a new file and then open it for

WRITE operations.

'Append' Same function that 'Write' but if the file

is already existing, then append the new data.

When you have finished to write this file you can close it.  The  close

operation  can  be  performed  by  a  new OPENFILE statement on the same I/O

chanel, or by the CLOSEFILE statement.

The CLOSEFILE syntax is :

CLOSEFILE [ :  <cchannel> ] ;

The default I/O channel is always 1.

To write anything to the opened file,  we  use  the  WRITE  or  WRITELN

statements as :

WRITE [ :<cchannel> ] <anything(s)> ;

WRITELN [ :<cchannel> ] <anything(s)> ;

Each call of WRITELN write one line on the output file,  but  WRITE  do

not end the current line.

To read anything from the opened file, we use the READ statement as :

READ [ :<cchannel> ] <anything(s)> ;

The syntax of the WRITE statement is exactly the same as  WRITEMSG  and

DISPLAY statements and the syntax of the WRITE statement is exactly the same

as REPLY statements.

MACRO FACILITIES IN THE MXD LANGUAGE.                                  Page IX-9

The $STATUS predefined identifier can be used to know the result of the

last  file  operation  (INCLUDE,  CHAINE,  MACROCALL, OPENFILE, READ, WRITE,

CLOSEFILE).

The $STATUS values are:

0             Succesfull operation.

-1             End of file reached (READ operation) or

Not existing file (OPENFILE statement).

-other values  System error code for PASCAL open.

CHAPTER X

THE LEAST SQUARE PROGRAM MXDLSQ.

The MXDLSQ program is the Least  Square  fitting  program  of  the  MXD

system.   This program perform all crystallographic computations as asked in

the source MXD data file(s).  The user formulae are derivated  by  a  formal

process.  The physical used formulae are given in the Theoritical chapter of

this manual and the aim of the present once is to indicated the use of  each

option as defined in the MXDLSQ code.

X.1  THE SPECIFICATIONS OPTIONS.

These options are used to specify the wanted number of  cycle,  exclude

or reject some data from the data collections and/or modifies the refinement

processing.

1.  NCYCLE To set the wanted number of reffinement cycle.

Format :  OPTION(NCYCLE) = <c#nbcycle>;

2.  MXCATEG To work only with the observation  beyong  any  category  number

equal or lower than the given category number (Category rejection).

FORMAT :  OPTION(MXCATEG) = <c#maxcateg>; { default 63 }

All reflection with a catogory number greater than maxcateg must be

rejected  from  the  reffinement.   The  category number is in the range

[1..63]

3.  MXSITHSL To give a upper limit for 1/2d of used reflection (Rejection).

THE LEAST SQUARE PROGRAM MXDLSQ.                                        Page X-2

FORMAT :  OPTION(MXSITHSL) = <c#upper_value>; { default no limit }

This option can be used to do a rejection based  on  the  value  of

sin(theta) / Lambda = 1/2d.  If used all reflection with an 1/2d greater

than the specified upper_value is rejected from the fitting.

4.  MISITHSL To give a lower limit for 1/2d of used reflection (Rejection).

FORMAT :  OPTION(MISITHSL) = <c#lower_value>; { default 0.0 }

This option can be used to reject any reflection with a 1/2d  lower

than  the  specified  lower_value.  It can be combined with the MXSITHSL

option to select a 1/2d range.

5.  HKLLIM To fit only with the reflection inside a particular index range.

FORMAT :  OPTION(HKLLIM) = <c#hmin>, <c#hmax>

[, <c#kmin>, <c#kmax>, <c#lmin>, <c#lmax> ];

{ default is no limit range }

You can reject any reflection not in the specified range  of  index

H, K, L.  If only one range is specified it is applied at H K and L.  If

the maximum is given only the minimum is - the maximum.

6.  REJECT  To  set  an  automatic  rejection   to   the   agreement   ratio

Delta / Sigma .

FORMAT :  OPTION(REJECT) = <c#reject_lim>; { default:  no rejection

}

When specified during each least square cycle, any reflection  with

a  Delta / Sigma agreement ratio (delta = $OBS - $CALC) greater than the

specified reject_lim value is  rejected  for  this  cycle.   This  is  a

temporary  rejection, if at the next cycle the ratio is decreasing below

this limit, then the reflection is used.

7.  VARROT To set a variable rotation increment  accross  the  least  square

diagonal blocks.

THE LEAST SQUARE PROGRAM MXDLSQ.                                        Page X-3

FORMAT :  OPTION(VARROT) = <c#incr>; { default is 0 }

If a positive increment incr is specified the last incr variable of

each diagonal block are shifted in the next diagonal block, and the incr

variable of the last diagonal block are pushed in the first one's.

8.  SAVECYCLE To do a SAVE file at each  cycle,  the  save  file  names  are

"CYCLEn.MXDSAVE"  where  n is the cycle number.  These files can be used

as the normal save file created by the  SAVE  statement  by  an  INCLUDE

statement.  The open mode of these file is the unknown mode, so the file

are superseded when they are already existing.

FORMAT :  OPTION(SAVECYCLE) = { YES | NO }; { default YES }

9.  MAXSING To set a limit of allowed singularity during  the  inversion  of

least  square  matrix.   The  default  is the half of the total variable

number.

FORMAT :  OPTION(MAXSING) = <c#max_nbs>; { default Nvar / 2 }

By default MXDLSQ accepts  the  singularity  of  the  least  square

matrix.In  the singularity case, MXDLSQ clear all correlation factors of

the singular variable and set to one (1.0)  the  corresponding  diagonal

term.   the related variable is flagged as a "LOCKED" variable and it is

fixed for this cycle.  If during a cycle, the number of  singularity  is

greater  than  the  current  value  of  max_nbs,  then  MXDLSQ  stop the

reffinement.

10.  MINDIAG To set an artificial singularity.

FORMAT :  OPTION(MINDIAG) + <c#minpivot>; {default 0.0}

If, during the matrix inversion process  (by  Sholesky  method),  a

matrix pivot is lower than the specified value, it is considered as null

and an artificial singularity occurres.   This  option  can  be  a  good

method  to  get  an  acceptable  convergence  with  a  limited number of

observation.

X.2  THE LISTING OUTPUT OPTIONS.

The MXDLSQ option are in the following list :

THE LEAST SQUARE PROGRAM MXDLSQ.                                        Page X-4

1.  SHORTLST To enable or disable the short listing  mode  (the  default  is

enabled).   When  the  short listing mode is enabled, many other listing

options are discarbed ( as LISTHKL) and the LARGLINE is forced to NO.

Format :  OPTION(SHORTLST) = { YES | NO };

Default :  OPTION(SHORTLST) = YES;

2.  LISTHKL To drive the listing of the reflections.

Format :  OPTION(LISTHKL) = {YES | NO [, <c#rejlst>] };

Default option :  OPTION(LISTHKL) = NO;

Enable(YES) or  Disable(NO)  the  reflection  listing  output.   If

Enabled  and  if  rejlst is specified then only the reflections computed

with the delta/sigma > rejlst are listed out.  In this last case the hkl

reflections inside a reflection set are never listed except the last.

3.  LISTRES To enable or disable the listing out  of  intermediary  variable

results.

Format :  OPTION(LISTRES) = { YES | NO };{ the default is YES }

If YES is given (default) the new, old, change and  sigma  of  each

variable  is  listed  out for each cycle.  If No is specified, then only

the final value and sigma are listed out after the last cycle.

4.  LISTMAT To output on the listing all the correlation matrix.

Format :  OPTION(LISTMAT) = {YES | NO}; {default NO => Disable}

Enable the listing of the final correlation  matrix  if  YES.   The

correlation  matrix output can be very large, so for the large number of

variable (by example more than 30 variables) it can be more efficient to

use the MXCORREL option.

5.  DSPLPAR To display until four (4) variable parameters in the  reflection

listing.

THE LEAST SQUARE PROGRAM MXDLSQ.                                        Page X-5

FORMAT :  OPTION(DSPLPAR) = PARAMREF(<param_ident>) [,...];

Param_ident must be the name (identifier) of a Variable  Parameter.

The  standard  function  PARAMREF  is used to get the sequence number of

this Variable Parameter Identifier.  Until four (4) Variable  Parameters

can  be  specified.   If  there  are  more, then only last four would be

displayed.  This option is usefull to known any parameter value that  is

computed  for  each  reflection  as  an  extinction  coefficient,  or an

analytic form factor...

6.  STRINI To output a structure description before  the  first  reffinement

cycle.

FORMAT :  OPTION(STRINI) = {YES | NO}; {default NO => Disable}

If this option is enabled then MXDLSQ output a description  of  the

structure  with  all  defined  asymmetric  atoms, all special atoms with

there different modulation and/or magnetic moments.

7.  STREND To output a structure description  after  the  final  reffinement

cycle.

FORMAT :  OPTION(STREND) = {YES | NO}; {default NO => Disable}

This option has the same function that  the  option  STRINI  except

that this is the final structure that can be listed out.

8.  LISTSYM To output the list of symtry matrix(s) in the initialize time.

FORMAT :  OPTION(LISTSYM) = {YES | NO}; {default NO => Disable}

This option can be used to display on the listing the list  of  all

symtry operators ( origine center eqivalent are not output).

9.  LISTVER To enable or disable the listing out of partial R factors.

Format :  OPTION(LISTRES) = { YES | NO };{ the default is NO }

THE LEAST SQUARE PROGRAM MXDLSQ.                                        Page X-6

If YES is given the R factor of each  data  collection  are  listed

during  the intermediary cycles.  But same if NO (the default) is given,

these partial R factors has listed out at the end cycle.

10.  MXCORREL To ask to MXDLSQ to display any correlation factor with a value

greater than the specified value.

FORMAT :  OPTION(MXCORREL) = <c#max_corr>; { default is 1.0 }

This option is used to output on the listing  during  final  cycle,

all  correlation  factor greater than the specified value.  A often used

max_corr value is 0.65.

11.  DSPLFMAG To display the projected magnetic structure factor (FM) of each

reflection.

FORMAT :  OPTION(DSPLFMAG) = {YES | NO }; { default NO }

When used, this option inserts  the  projected  magnetic  structure

factor  in  the  reflection  listing.  If enabled all DSPLPAR option are

ignored.

12.  NBSELIN To display the select number in the data column or by message.

The YES value enables the output of SELNB in a LISTHKL output as  a

column

number.  If NO value is specified, then each change of SELNB insert

a message in the LISTHKL output.

FORMAT :  OPTION(NBSELIN) = { YES | NO }; { default yes }

13.  LISTPAR To Enable the list of all known variable parameters.

FORMAT :  OPTION(LISTPAR) = { YES | NO }; { default yes }

If this option  is  set  (default),  all  the  variable  parameters

(except  those  with  a name begining by the character #) will be listed

out at the end of run, with there last values  and  there  last  related

sigma (computed to use the current correlation matrix).

THE LEAST SQUARE PROGRAM MXDLSQ.                                        Page X-7

14.  FMAGDISP  To  display  the  magnetic  structure  factor  (Fm)  of   each

reflection.

FORMAT :  OPTION(FMAGDISP) = {YES | NO}; { default NO }

When used, this option inserts the unprojected  magnetic  structure

factor  in  the  reflection  listing.  If enabled all DSPLPAR option are

ignored.  This option is not compatible with the DSPLFMAG option.

15.  LIFEDISPL is a future life display option.   its  use  is  reserved  and

without garanty.

X.3  OUTPUT OPTIONS TO GO TO OTHER PROGRAMS.

1.  UPDREF To output a binary file of final computed structure  factors  (to

use as Fourier input file) for each data collection.

FORMAT :  OPTION(UPDREF) = {YES | NO}; {default NO => Disable}

This option is given to use with a Fourier program.  In VAX/VMS MXD

implementations some Fortran programs in MXDLIB:  are given to prepare a

file for SDP or MK3 crystallographic system that use these files.

The fortran program MXDCAM source in the disk area  MXDLIB  can  be

take  as  a  good  example  to get informations from the MXD binary data

files.  For each  data  collection  MXDnnn.BDA  is  the  data  file  and

MXDnnn.BCF  is  the computed structure factor file where nnn is the data

collection number which start from 000.

2.  MK3OUT To enable the creation of a computed reflection list at  the  end

of this run in a MK3 compatible format to perform spin density map.  The

file  name  is  'MXDnnn.MK3"  where  nnn  start  from  000  for  and  is

incremented by 1 for each data collection.

FORMAT :  OPTION(MK3OUT) = {YES | NO} [, Hx, Hy, Hz  ];{default  NO

and  0  0 1} The (Hx, Hy, Hz) vector is used as magnetic direction.  The

magnetic structure factor is projected on this vector  to  give  the  FM

(magnitude) and PH (phase) output on the created file.

THE LEAST SQUARE PROGRAM MXDLSQ.                                        Page X-8

The file has the following format :

|C  fortran source

|        READ(1,500) IH, IK, IL, FM, PH, DOBS, DELTA

| 500    FORMAT(3I4,2F12.5,F10.3,F16.5,F12.5)

Where IH, IK, IL denote the reflection index, Fm  is  the  magnetic

structure  factor componante along the applied magnetic field, and Ph is

the related  phase,  DOBS  is  the  observation  and  DELTA  is  DOBS  -

computedvalue.

3.  ATOMOUT To output an atom coordinate list for other program in the  file

"ATOM.DAT".   This  file  is  directly  compatible with the ORTEP (Carol

Johnson) crystallographic structure display program.

FORMAT :  OPTION(ATOMOUT) = { YES | NO }; { default NO }

4.  SAVEMAT To save the final correlation matrix at the end of run.

FORMAT :  OPTION(SAVEMAT) = { YES | NO}; {default is NO };

This option can  be  used  to  get  an  ASCII  file  with  all  the

correlation matrix and sigma of variable information.

CHAPTER XI

VAX/VMS IMPLEMENTATION OF MXD.

In   this   chapter    we    present    the    particularity    of    the

VAX/VMS (Digital Equipment Coorporation) of MXD.

XI.1  MXD INSTALLATION.

This file is a basic guide to instal the MXD software.

XI.1.1  The First MXD Installation Is Performed In Steps As Below :

1/  You  must  get a  directory  to put in all  MXD files.

This  can be  a particular directory  in  your private

disk  area  or  a  more  common  directory  disk area.

In this last one, you must contact your system manager

to  get  this  disk area  with the common  user  read,

execute, write  and delete  access  for you  and  read

and execute access for all the MXD users.

2/  You must logged in and  enter in  this particular disk

disk area by a command as ( example ) :

$ SET DEFAULT DUA3:[SOFTWARELIB.CRYSTALLO.MXD]

3/  You must mount the  MXD kit magtape on a magtape drive

by a  VMS  mount command. If  the magtape name is MUA0

the command is:

$ MOUNT MUA0:/FOR

After, you must read this magtape  with the VMS BACKUP

utility by the command :

VAX/VMS IMPLEMENTATION OF MXD.                                         Page XI-2

$ BACKUP/REW MUA0:MXDSET/SAVE []/NEW

At the end of BACKUP  you can dismount  the magtape by

the next VMS command

$ DISMOUNT MUA0:

and remove the magtape kit of the drive.

4/  Now  you  run  the  command file  MXDINSTAL  with  the

parameter CONTINUE as this :

$ @MXDINSTAL CONTINUE

This command file  performs all appropriate actions to

complete the MXD software installation. The  generated

dialogue  give  the  opportunity   to verify   the MXD

installation  by the  execution  of the MXDIVP command

file ( IVP as Installation Verification Product ).

XI.1.2  TO Upgrade From A Previous Version 3.4 Or Above.

The upgrade from a previous MXD version is performed by the  VMS  command

file  MXDINSTAL  if  your  actual MXD version is the version V3.4E or any more

recently version.  For all old version ( with a version number  below  3.4E  )

you must proceed as for the first installation except the step   1.

The Upgrade of MXD version is performed by the command :

$ @MXDINSTAL

The command file performs all appropriate  actions  to  upgrade  the  MXD

software  installation.  The generated dialogue give the opportunity to verify

the MXD installation by the execution of the MXDIVP  command  file  (  IVP  as

Installation Verification Product ).

The MXDINSTAL command file performs the mount, backup and dismount of the

magtape  and  all  other  appropriate  action  as:  replaces all old files and

purges the old versions.  In some case  in  can  be  advisable  to  mount  the

magtape  before to activate the procedure.  In this situation you must use the

following DCL commands :

VAX/VMS IMPLEMENTATION OF MXD.                                         Page XI-3

$ MOUNT MUA0:/FOR

$ @MXDINSTAL

In this case the MXDINSTAL procedure do not dismount the magtape  on  its

complexion.

XI.2  VAX/VMS MXD COMMANDS.

A limited set of command is defined by the file "MXDLIB:MXDDEF.COM".

XI.2.1  The Least Square Command MLS.

This is the main MXD command to work with MXD.

The MLS command syntaxe is :

$ MLS <primary_file_name> [ <listing_devicename> [ <savedevicename> ] ]

The first parameter, that must be present, is the primary MXD source file

name  that  you have created (with EDT(Edit ), TPU ...  and so on ...  ).  The

second  parameter  is  the  device  where  the  listing  files  "MXD.LIS"  and

"MXDLSQ.LIS"  must  be written.  If it is not specified or if you specify "*",

then the controlling terminal "TT:" is used.  The last parameter  is  used  to

specify  if  you  want  use  a previously created file where the last variable

values are saved (by MXD).  The save device name is the physical name that you

want  to  associate  to the logical device "IN:".  By default before the first

use of the MLS command, the logical device "IN:" is defined as the null device

"NL:".

VAX/VMS IMPLEMENTATION OF MXD.                                         Page XI-4

After for the next use of  the  MLS  command  the  "IN:"  logical  device

definition is changed only if the third MLS parameter is specified.

To use this Third parameter the INCLUDE statement in the  MXD  file  must

use the "IN:" device as this :

INCLUDE 'IN:filenameofsavefile';

XI.2.2  The MXD Command.

They have the forms :

$ MXD [ <primary_file_name> ]

If the file is not specified, then MXDCMP  is  called  in  conversational

mode  and  prompt you to get a legal MXD command.  You can return to DCL level

to use the END; or ENDFILE; commands or the end of file character Z.  This MXD

command  usage  can  be  usefull to do some small computings in conversational

mode as this :

MXD> DISPLAY sin(30);

Output the sinus of 30 degrees on the terminal in floatting format.

All the legal MXD command can be use in this mode.

If a file is specified, Then MXDCMP proceed this file as the command  MLS

except two things :

VAX/VMS IMPLEMENTATION OF MXD.                                         Page XI-5

The MXD.INI file is never deleted by the MXD command and you must  delete

it after by the command MXDDEL.

The MXDLSQ program is not called.

A typical application of  the  MXD  command  is  to  create  a  spherical

absorbation  table for a given spherical single crystal by the use of the file

"MXDLIB:ABSORB.MXD" that use the DISPLAY REPLAY statements and the output on a

ASCII  file  to  create  the  file  "ABSTAB.MXD".   This file can be used in a

primary MXD file (by INCLUDE 'ABSTAB.MXD'; statement) to get  the  appropriate

absorbation and average way (for extinction) tables for your sample.

XI.2.3  The MXDDEL Statement.

You must use it each time where the MLS or the MXD  (with  specified  MXD

file) are canceled by a VMS error to delete the trailing "MXD.INI" file(s).

$ MXDDEL

The MXDDEL definition is :

$ MXDDEL == "DELETE MXD.INI;*"

APPENDIX A

ERROR NUMBER SUMMARY

The general form of an MXD Data Compiler error message is:

[FATAL] ERROR <mdn> # <nb> AT "<stat>" STATEMENT # <cpt>

IN THE FILE : <filespecif>

AT THE LINE # <linenb>

The error number <nb> is associated  at  the  summary  listed

below, the "-" sign flags the always FATAL errors indicated by the

word FATAL in the error message.  <cpt> is the  current  statement

count at <linenb> the source line number in the input file defined

by the file specification <filespecif>.  <stat> is the name of the

current  parsed  statement.  The Item <mdn> is a briev form of the

error detector program modul name.

A.1  Core Allocation and Input Stream Errors

--------------------------------------------

1:    Too long input data line(max. = 132 characters).

-2:    Heap allocation overflow.

-3:    Context stack overflow.

4:    Array parameter interpolation (implied) reference overflow.

-5:    Operand stack overflow.

-6:    INTERNAL: Operand stack underflow.

-7:    Operator stack overflow.

-8:    Identifier scope stack overflow.

11:    Exponent overflow (on read constante number).

12:    Constant string too long (max. = 132 characters).

13:    Illegal character.

14:    Data Stream error: "+" or "-" expected.

15:    Data Stream error: "," or ";" expected.

ERROR NUMBER SUMMARY                                                    Page A-2

16:    Data Stream error: second polarisation sign expected.

17:    Illegal in Data Stream.

-18:    Illegal INCLUDE/CHAINE syntaxe.

-19:    Data field list overflow(max. = 32).

A.2 Computing Errors.

---------------------

21:    Integer overflow.

22:    String length overflow(max. = 132 characters).

23:    Out of range ASIN/ACOS parameter.

-24:    INTERNAL.Illegal interpolation.

25:    Zero length array parameter for interpolation.

26:    Array parameter with length less than 4.

27:    Zero divide operation.

28:    Real power of a negative number.

29:    Logarithme of a negative argument.

30:    Square root of a negative argument.

31:    Out of range item field index.

A.3 Syntaxe Errors.

-------------------

51:    Undeclared identifier.

52:    String parameter name expected.

-53:    Binary operator, types and operator cannot match.

-54:    Two constant parameters expected for a binary operator.

-55:    Unary operator not allowed with this type of operand.

-56:    Constant parameter expected for this unary operator.

57:    Numeric expression expected.

58:    Numeric constant expression expected.

59:    Variable identifier expected.

60:    String constant parameter expected.

61:    Identifier expected.

62:    Illegal use of a reserved keyword.

63:    Identifier declared twice.

64:    Parameter type cannot match in an assignement statement.

65:    Initial value (to define the parameter type) expected.

66:    Undefined statement.

67:    Bad use of an unary operator.

68:    Unexpected END/ENDIF/ENDMACRO/ELSE//UNTIL keyword.

69:    An array parameter (as form factor) was required.

71:    "THEN" is expected in IF statement.

-72:    "UNTIL <const-expr> ;" was expected in a REPEAT statement.

73:    A string or numeric param identifier was expected.

74:    A string or numeric constant expression was expected.

ERROR NUMBER SUMMARY                                                    Page A-3

75:    A variable identifier was expected.

-76:    External or Physical statement not allowed in openfile

mode.

-77:    Unexpected End-of-file in this context.

78:    Value expected for  for a parameter declaration.

-79:    Illegal assignassion to an existing variable parameter.

81:    ")" expected.

82:    "(" expected.

83:    "]" expected.

84:    "[" expected.

85:    "," expected.

86:    ";" expected.

87:    "," or ";" expected.

-88:    INTERNAL. Undefined type.

89:    "+" or "-" expected.

90:    "=" expected.

91:    "=" or "," expected.

-93:    Illegal ENDFILE statement in a macro.

-94:    INCLUDE/CHAINE illegal in a macro.

95:    Macro identifier expected.

96:    Undeclared macro.

-97:    Macro parameter defined twice.

-98:    Macro declared twice.

99:    Option identifier expected.

101:    Undeclared item.

102:    Item declared twice.

103:    Minimum of one physical field  definition expected for an

item.

104:    Some unit cell data expected.

-105:    Negative or zero unit cell volume.

-106:    Rhombohedral unit cell expected.

107:    Undefined lattice type.

-111:    Too many macroname in one MACROCALL statement(max = 64.).

-112:    Some macro(s) are not find in the current macro library.

151:    data predefined identifier expected.

152:    Ambiguous data nature definition.

-153:    Predefined identifier already used.

-154:    Undefined data directive.

-155:    Identifier found in a data stream line.

-156:    Neutron polarization specification unexpected.

157:    Negative multiplicity.

158:    External Data Collection not exists.

-171:    Illegal use of CLRDATA statement after a DATA declaration.

-201:    Illegal value for an I/O channel.

-202:    OPENFILE mode specification was expected.

-203:    Illegal OPENFILE mode specification.

ERROR NUMBER SUMMARY                                                    Page A-4

-204:    User file was not opened for the READ operation.

-205:    User file was not opened for the WRITE operation.

-206:    Cannot open the file.

-441:    A declared function was expected.

-442:    The function parameter list do not matchs.

-444:    SUMM index identifier cannot be created do to an error.

-999:    An ERROR statement was executed.

APPENDIX B

HOW WRITE MXD DATA AS STANDARD.

The MXD input data file can be write with many various method, but it can

be  a  good  pratic  to  follow  a  standard  presentation  to make it easy to

understand.

As it is possible,  the  user  is  strongly  encouraged  to  follow  this

statement order :

Constant definitions if used.

Macro call and macro definitions if used.

MXDLSQ options as NCYCLE, LISTHKL ... and so on.

Variable declarations with LIMITS and LSQBLOCK directives.

Parameters declarations if used.

Atoms declarations.

Moments or MDSDSP declarations.

Related data parameters definitions.

Experimental Data collection decalration.

APPENDIX C

LEGAL EXPRESSION OPERATORS AND STANDARD FUNCTION.

C.1  OPERATORS FOR ALL CONSTANT EXPRESSIONS.

They can be applied at any numeric or string expressions.

<sc#e1>  = <sc#e2>       test if equal.

<sc#e1> <> <sc#e2>       test if not-equal.

<sc#e1>  > <sc#e2>       test if greater than.

<sc#e1> >= <sc#e2>       test if greater than or equal.

<sc#e1> <= <sc#e2>       test if less than or equal.

<sc#e1>  < <sc#e2>       test if less than.

C.2  OPERATORS FOR NUMERIC ALL EXPRESSIONS.

<#e1>  + <#e2>         addition operator.

<#e1>  - <#e2>         substract operator.

<#e1>  * <#e2>         multiply operator.

<#e1>  / <#e2>         divide operator.

<#e1>  ^ <#e2>

or <#e1>  ** <#e2>        exponential operator.

- <#e>            unary minus operator.

+ <#e>            unary plus operator.

C.3  OPERATOR FOR STRING EXPRESSIONS.

<s#e1>!! <s#e2>        concatenation operator.

LEGAL EXPRESSION OPERATORS AND STANDARD FUNCTION.                       Page C-2

C.4  FUNCTIONS FOR VARIABLE EXPRESSIONS.

INTSEL(<e sel>,<e 0>[,<e 1>,...<e 31>]) select one value function

The selected expression is the

<e sel> expression.

SELECT(<#e0>,<#e1>,<#e2>,<#e3>)  select one value function

The selected expression is the

SELNB (defined in DATA) expression.

SUMHKL(<e x>)                 return the sum of all valid hkl in

data if the parameter expression.

C.5  FUNCTIONS FOR ALL NUMERIC EXPRESSIONS.

ROUND(<#e>)           the nearest integer value of <c#e>.

MODULO(<#e1>,< e2>)           return the modulo of <#e1> by <#e2>

SIN(<#e>)           sinus.

COS(<#e>)           cosinus.

TAN(<#e>)           tangent.

LOG(<#e>)           natural logarithme.

EXP(<#e>)           exponential.

ASIN(<#e>)           arcsin.

ACOS(<#e>)           arccosinus.

ATAN(<#e>)           arctangent.

TANH(<#e>)           hyperbolic tangent.

SQRT(<#e>)           square-root.

ATAN(<#eb>,<#ea>)   phasis of the complex number ea + i* eb.

ABS(<#e>)           magnetude of <c#e>.

ROUND(<#e>)           the nearest integer value of <c#e>.

C.6  OPERATORS FOR ALL NUMERIC EXPRESSIONS.

<#e1> ! <#e2>        logical inclusive or.

<#e1> & <#e2>       logical and.

C.7  FUNCTIONS FOR NUMERIC CONSTANT EXPRESSION.

NUMBER(<s#e>)         convertion the string in integer number.

INDEX(<s#e1>,<s#e2>)  give the position of the first recurence

of <s#e2> in <s#e1> if found, and 0 else.

LENGTH(<s#e>)         give the length in character of <s#e>.

LEGAL EXPRESSION OPERATORS AND STANDARD FUNCTION.                       Page C-3

C.8  OPERATORS AND FUNCTIONS FOR STRING EXPRESSION.

<s#e1> !! <s#e2>       string concatenation operator.

STRING(<c#e1>)        conversion of the nearest integer of

<c#e1> in numeric string.

STRING(<c#e1>,<c#e2>) same, but complet the string to <c#e2>

digits with add some "0" at left.

SUBSTR(<s#e>,<c#e1>,<c#e2>) extrac a substring begin at the

<c#e1> th. character of <s#e> and with

<c#e2> length.

if <c#e2> is omitted then the substring

is limited at the and of the string <s#e>.

C.9  THE PRE-DECLARED VARIABLE PARAMETERS.

MXD has a variety of pre-declared variable identifiers, The Variable terms  is

used  to  tell that the actual value is undefined.  These identifiers name can

be used anywhere a variable expression can  be  used  except  for  the  symbol

$CALC.

All these parameters are related with the current reflection.

$H,$K,$L         Reciprocal lattice vector H coordinates for the

current reflection has given in DATA statement.

$RH,$RK,$RL      Reciprocal lattice vector H in real coordinates

in 1/Angstroem(s).

$SH,$SK,$SL      Symmetry transformed reciprocal lattice vector H in

real coordinates in 1/Angstroem(s). Can be used to

define any anisotropic form factor or anharmonic

thermal vibration factor.

$HH,$KK,$LL      Scattering vector coordinates in the reciprocal space

in 1/angstroem(s). This vector is given by the

formula : h = H + NQ * q where q denotes the related

reflection wave vector.

$SITHSL          SIN(THETA)/LAMBDA = 1/(2*d) = magnitude of the h vector.

$QX,$QY,$QZ      The coordinates of q wave vector in 1/angstroem(s).

$HX,$HY,$HZ      The magnetic field components for neutron polarization.

$OBS             The observed data as given in DATA statement.

$SIG             The sigma value as given in the DATA statement.

$WEIGHT          The weight as given in the DATA statement.

$CALC            The value of the current F or F**2 computed structure

factor. This identifier can be only used in a data scale

expression or a related parameter.

$FN2             The squared crystallographic structure factor.

$FM2             The squared and projected magnetic structure factor.

$F2POLA          Partial polarized neutron intensity (I++,I--,I-+ or I+-).

$FNR, $FNI       Real and imaginary parts of the atomic structure factor.

LEGAL EXPRESSION OPERATORS AND STANDARD FUNCTION.                       Page C-4

$FMXR,$FMXI )

$FMYR,$FMYI  )   Components of the real and imaginary parts of the

$FMZR,$FMZI )    unprojected magnetic structure factor.

$LCHI2, $CCHI2   The last and the current squared goodness of Fit.

$LMAXF, $CMAXF   The last and the current likehood function.

APPENDIX D

THE STANDARD MACRO LIBRARIES.

The standard macro libraries is a  divide  in  two  sets.   The  standard

library "MXDLIB.MXL" for various purpose, and the secondary extinction library

"EXTINC.MXL" .

In the VAX/VMS implementation these two libraries are located on the  MXD

disk area "MXDLIB:" .

All the macro libraries can be printed since that are text files.

D.1  THE GENERAL PURPOSE MACRO LIBRARY "MXDLIB.MXL" .

This library is always selected to load the required  macro.   The  macro

can be loaded by two methods :

The standard method by the use of the statement MACROCALL (Chapter IX) or

by the use of the predefined macro $.  The macro $ is defined in the "MXD.OPT"

file that is included by the "MXD.INI" file (created by the MLS command).

The list of the $ macro is :

MACRO $ P0,P1,P2,P3,P4,P5,P6,P7,P8,P9,P10,P11,P12,P13,P14,P15;

MACROCALL P0;

P0 P1,P2,P3,P4,P5,P6,P7,P8,P9,P10,P11,P12,P13,P14,P15;

PURGE P0;

ENDMACRO;

THE STANDARD MACRO LIBRARIES.                                           Page D-2

By the use of the macro $ the use of a standard macro is performed by :

$ <macroname> [ <listofmacroparameters> ] ;

The next examples use this macro when the macro is used one unique  time.

If  the  macro  has  many  call (HELMOM or HELDSP), then it is better to use a

statement as :

MACROCALL HELMOM, HELDSP; { Load HELMOM and HELDSP }

D.1.1  The HELMOM Macro For Helimagnetic Moment Description.

To creat a complex magnetic moment M = u-i*v with :

Mu and Mv denote the magnitudes of u and v,

Frm denotes magnetic form factor,

Om, Chi, Phi are Eulerian angles,

Al is the ellipse circulation phasis.

mnam  will be the name of the new magnetic moment,

anam  is the name of the related atom,

qnam  is the name of the related wave vector.

The Eularian angle are defined as to be zero when u and v are respectively

parallel to the X and Y axis of the working space.

Then the MXD statement creates an helimagnetic moment are in this

example :

HELMOM 'MQ1FE3', 'FE3', 'Q1', Om, Chi, Phi, Mu, Mv, Al;

The extremity of the magnetic moment MQ1FE3 describe an ellipse  when  we

change of the unit cell with the variation of the scalar product :

----->   -->

r(FE3) * Q1

D.1.2  The HELDSP Macro For Modulated Displacement Description.

To creat a complex magnetic displacment D = u-i*v with :

Du and Dv denote the magnitudes of u and v,

THE STANDARD MACRO LIBRARIES.                                           Page D-3

Om, Chi, Phi are Eulerian angles,

Al is the ellipse circulation phasis.

dnam  will be the name of the new displacment,

anam  is the name of the related atom,

qnam  is the name of the related wave vector.

The Eularian angle are defined as to be zero when u and v are respectively

parallel to the X and Y axis of the working space.

Then the MXD statement creates an harmonic displacment are in

this example :

HELDSP 'DQ1FE3', 'U1', 'Q1', Om, Chi, Phi, Du, Dv, Al;

The extremity of the displacment vector DQ1FE3 describe an  ellipse  when

we change of the unit cell with the variation of the scalar product

----->   -->

r(U1) * Q1

D.1.3  Le Page And E.J.  Gabe Extinction Correction.

This correction of the secondary extinction can be give good result  when

the  extinction is not to large.  The correction computing is performed by the

statement :

$ E$PGNA;

This  macro  define  the  variables   E$GBETA   the   extinction   mosaic

coefficient, and E$X the ratio of the normal cinematic crystal.

The parameter E$Y is defined as the extinction correction coefficient  to

applied to the squared structure factor by the use of the DATA statement.

Example :

DATA( <name>, ( <scalefac.>, E$Y ), ... ) ... ;

THE STANDARD MACRO LIBRARIES.                                           Page D-4

Note :  The E$PGNA macro is $FN2 ( squared  nuclear  structure  factor  )

based correction, then it is not appropriate for an magnetic diffraction.

D.1.4  SHEL-X Extinction Correction.

This correction of the secondary  extinction  is  given  to  be  able  to

compare  with  the  SHEL-X  refinement  results.   The correction computing is

performed by the statement :

PARAM $LAMBDA = 0.96; { define the used wavelength }

$ E$SHELL;            { define E$Y }

This macro define the variable E$X  the  extinction  mosaic  coefficient.

The  parameter  E$Y  is  defined  as  the extinction correction coefficient to

applied to the squared structure factor by the use of the DATA statement.

As for the E$PGNA macro, E$SHELL is $FN2 based and cannot be applied to a

magnetic diffraction.

D.1.5  Cooper And Roose Formula For A Cylinder Absorption.

When we refine from powder is not always possible to know the  absorption

because  it  is  difficult  to  evaluate the powder density (as for example in

metallic hydride powder).   So  it  is  interresting  to  fit  the  absorption

coefficient  directly.   If  the absorption is not to large, it is possible by

the use of the Cooper and Roose formula.  The cylinder is the typical form  of

the neutron powder sample.  To use it :

$ ABS$CYL;

The A$MUER variable is created, and the $ABS parameter is defined  to  be

the absorption coefficient that can be used as a scale correction factor.

Example :

DATA( <name>, <scalefac.>*$ABS, ... ) ... ;

THE STANDARD MACRO LIBRARIES.                                           Page D-5

D.2  THE P.  BECKER AND P.  COPPENS ( LINEX LIKE ) EXTINCTION CORRECTION.

The macro EX$BECCOP give all  the  extinction  correction  as  the  LINEX

program.  The use follow the next model :

MACROLIB 'MXDLIB:EXTINC.MXL'; { set the extinction library }

MACROCALL EX$BECCOP;          { load the Becker and Coopens macro }

{ before to call the macro always define the wavelength }

PARAM $LAMBDA = 0.96;         { define the wavelength (Angstoem) }

{ not always required }

{ for spherical crystal }

INCLUDE 'ABSTAB.MXD';         { get the definitions of $ABS and E$TMU

}

{ for non spherical crystal }

FFASSIGN E$TMU:-1             { assign the last column   at E$TMU }

{ E$TMU will be read

by the DATA statement }

EX$BECCOP <irad>, <iso>, <ityp>, <idist>,

[ <r>, <mu>, [ <thtm>, [ <UBMATRIX> ] ] ] ;

where :

irad is   0  for neutron data,

1  for x-ray data.

iso  is   0  for isotrope mode,

1  for anisotrope type Coppens-Hamilton,

2  for anisotrope type Thornley-Nelmes.

ityp is   1  for a crystal type I,

2  for crystal type II,

3  for the general case.

idist is the distribution type used only when ityp =1 or 3,

0  for Gaussien distribution,

1  for lorentzien distribution.

The following parameters are not always required.

The two next parameter are used only when E$TMU is not defined.

r     is the sherical sample rayon if used.

mu    is the linear absorption coefficient if used.

thtm is used only in the x-ray case.

thtm  is the monochromator Bragg angle in degrees or 0.0

The UB matrix is used only in anisotropic cases.

It is defined as this :

THE STANDARD MACRO LIBRARIES.                                           Page D-6

Rc  =  UB * Rg

Where, when omega = chi = phi = 0 (Goniometer angle at 0) :

the Rg vector is explain in the basis :

X // to incident beam and in the same direction,

Z is vertical,

Y = Z o X (vectorial product).

Rg is the vector attached to the goniometer head and

rc the same vector attached to the direct unit cell.

If the E$TMU symbol is not already defined EX$BECCOP use the  Cooper  and

Roose  correction  for  spherical  crystal,  then it must get the rayon of the

sample (in cm.) and the linear absorption coefficient (in cm-1).

If the sample is really a sphere, and the absoption is low this  solution

can  be  correct,  but  is  the absorption is important then the MXD procedure

ABSORB is  more  suitable.   In  this  last  case  the  user  creates  a  file

"ABSTAB.MXD"  by  the  command (VAX/VMS case) "MXD MXDLIB:ABSTAB" and includes

the file before the activation of the macro EX$BECCOP.   The  ABSTAB.MXd  file

defines  the symbol $ABS and E$TMU as the absorption and average of way in the

crystal.  If the crystal is not a  sphere  then  it  is  required  to  use  an

Gausse-Legendre  absorption  program  as  the  ARRANG  program  of the MK3 set

(Cambridge library).

When $F2 is not  defined  the  macro  EX$BECCOP  generate  a  $FN2  based

correction but the user can define $f2 before the macro activation as this :

PARAM $F2 = $FN2 + $FM2;

In this last case the extinction correction is performed for the  general

case where the reflection are mixed (nuclear and magnetic contributions).

For the polarized neutron computing we must define :

PARAM $F2 = $F2POLA;

THE STANDARD MACRO LIBRARIES.                                           Page D-7

Then the correction is computed on the  partial  intensity  contributions

I++, i--, I+- or I-+.

APPENDIX E

MXD DATA FILES STRUCTURES

MXD use the next following file to keep the data collections in a  easely

usable form.

MXD file name  created by,    used by,  internal form and use.

MXDINT.TMP     MXDCMP         MXDLSQ    ascii file

Reverse polish code and

directive for all application

program (MXDLSQ or other in

future).

MXDBDD.DDI     MXDCMP         MXDCMP    Pascal binary file,

list of the specification of

each defined data collection.

MXDnnn.BDA     MXDCMP         MXDLSQ    Pascal binary file,

MXDFOU*      One record by reflection line.

MXDSDP       nnn is the serial number of the

MXDCAM       data collection, that is starting

from 000.

MXDnnn.BCF     MXDLSQ         MXDFOU*   Pascal binary file,

MXDSDP       One record by computed refection

MXDCAM       line. It is used to get the

computed structure information

for the data collection write

in the file MXDnnn.BDA .

APPENDIX F

MXD INTERFACE WITH OTHER CRYSTALLOGRAPHIC SYSTEMS.

F.1  OUTPUT INTERFACES TO THE CAMBRIDGE LIBRARY MK3.

This interface are designed to perform  Fourier  (or  Fourier  difference

map) from the mxd binary datafiles.

This is the program in the file 'MXDLIB:MXDCAM.FOR'.

It read the selected MXDnnn.BDA and MXDnnn.BCF files to  write  an  ascii

file  for  the  FOURPL program of MK3 Cambridge library.  This is a Fortran 77

program that can be used as a model of interface.  It prompts the user to  get

the value of nnn number and ask if it is a F2 or F data collection.

Presently, the data information in the MXDINT file is not used.

F.2  INPUT INTERFACE FROM CAD4 GONIOMETER (ENRAF NONIUS 4-CIRCLES).

Built an ascii file form the CAD4 resulting data files.  The created file

can  be  "included"  directly  in the MXD DATA statement.  This is the program

'MXDLIB:REDMXD.FOR'.

F.3  INPUT INTERFACE FROM THE SDP (B.  FRENZ) CRYSTALLOGRAPHIC SYSTEM.

Fortran file:  'MXDLIB:SDPMXD.FOR'.

MXD INTERFACE WITH OTHER CRYSTALLOGRAPHIC SYSTEMS.                      Page F-2

Input:  Binary 'REFL.FOA' file.

Output:  Ascii file to "Included" in a MXD DATA Statement.

F.4  OUTPUT INTERFACE TO THE SDP (B.  FRENZ) CRYSTALLOGRAPHIC SYSTEM.

This program creat a REFL.FOA file with  the  standard  SDP  format  that

include  the  computed  and  observed  structure  factors.  They are scaled in

normalized structure factor units.  The magnetic contribution is duduced,  and

the  extinction  (or  other)  correction are applied, to get a correct Fourier

map.

Presently, the data information in the MXDINT file is not used, so MXDSDP

prompts the user to get the nnn number of the data collection.

Fortran file:  'MXDLIB:MXDSDP.FOR'.

Input:  MXDnnn.BDA and MXDnnn.BCF MXD binary files.

Output:  Binary 'REFL.FOA' file.

APPENDIX G

EQUIVALENT REFLECTIONS UTILITIES.

G.1  SORTING AN ASCII FILE IN INCREASING SIN(THETA)/LAMBDA.

The pascal program SORT is used to performs any theta sorting on a  ASCII

data  file,  the  output  form,  is  compatible with MXD and with the NONEQD2L

fortran program.

The input file must have one line per  reflection,  with  one  space  (or

tabulation)  between each number, the ordering of column is free.  It is the H

K L F2 and SG (reflection index h k l, squared structure factor, and sigma) in

any  order.   If  a  theta  column  is present the program can use it, else it

prompts the user to get the unit cell and then comput each theta Bragg angle.

The sort algorithme is very fast.  It use the binary memory tree and  can

sort  16500  reflection in two minutes by use of 1M bytes of memory in a micro

VAX II computer.

G.2  EQUIVALENT AVERAGE AND OR TWIN/MAGNETIC DOMAINS EXPANSSION.

The program NONEQD2L can be used to perform equivalent reflection average

on  a  given  symmetry  ponctual  group, and/or, expand input data for twin or

magnetic domain to a set of reflection  packets  (to  use  in  RAY  type  data

collection).

It use a previously theta sorted file (as created by  the  SORT  program)

and write a resulting file.  It use a parameter file set as this :

EQUIVALENT REFLECTIONS UTILITIES.                                       Page G-2

In the first line (free format).

Pmin, Icenter, Iout, Iexpe, Inmoy, Icoln, Imsg, Ils2

Pmin is the minimum of sigma to write in the resulting file.

.S

Icenter = 0/1 to donot/do the center operator active for the Friedel pair

[h,k,l] and [-h,-k,-l] same if the ponctuel group is not centered.

Iout = 0/1 to donot /do a full listing (file NONEQ.LIS)

Iexpe = 0/1 to donot/do the twin/domain reflection expansion to  generate

a data file for twin/magnetic domains fitting.

Inmoy = 0/1/2 to donot/donormalaverage/dofullreflectiongeneration.

if Inmoy is 0, there is no equivalent reflection generation.

if Inmoy is 1, NONEQD2L gives the normal hkl equivalent generation.

if Inmoy is 2, NONEQD2L generates all hkl equivalent reflection  but  one

reflexion  is  generate  for  each  symmetry  operator same when it is already

generated.  This mode is used with  Iexpe  =  1,  to  generate  each  magnetic

domains/twin reflection contribution.

Icoln is the total number column in the input file, the first column must

be IH, IK, IL, F2, SG, THETA, so Icoln must be in the range 6..13.

EQUIVALENT REFLECTIONS UTILITIES.                                       Page G-3

Imsg 0/1 to donotreplace/replace the  average  of  initial  sigma(s)  (on

founded  equivalent reflections) by the statistic error sigma deduced from the

equivalent reflections.

Ils2 0/1 to donot/do the LAMBDA/2  reflection  generation  (to  use  with

Iexpe = 1).

The second input line is (free format):

Nmat, Icenter where :

Nmat is the number of data symmetry operator matrix.

Icenter is 0/1 for notcentered/centered ponctual group.

The next lins must gives Nmat 3*3 integer matrixs.

The NONEQD2L program complet the ponctual group to get the full  operator

list, so the user is required to gives only the generator group elements.

The MXDLIB:P42MNM.GRP file is an example to perform the  average  of  the

equivalent  reflection  in the space group D4h/14, and the file MXDLIB:EP4.GRP

is an example to perform a domain expansion data file when the four domain can

be deduced by the ponctual group 4 from the others.

The resulting data file as the next following field :

NTWIN The domain number (starting from 0).  It is always 1 when  Iexpe  =

0.

EQUIVALENT REFLECTIONS UTILITIES.                                       Page G-4

IH, IK, IL the reflection indices.

The additional column values as  copied  from  the  input  first  related

reflection (if Icoln > 6).

ISMXD the sentinel for MXD to build the  set  of  reflection  lines.   If

Iexpe  is  0, then ISMXD = 1, else ISMXD is -1 for the contribution except for

the last one.

RF2, RSG, RTH are the resulting F2, Sg and theta.